API Reference for the USB2ANY Software Developer’s Kit

Version V2.8.2

Author: Randy Turchik
May 12, 2017

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Contents

Contents	2

Preface	5

1	Introduction	6

2	Overview of Hardware Design	6

3	Programming and Communications Protocol	6

4	API Summary	7

5	API Function Reference 	9

5.1	Controller Functions	9

5.1.1	u2aFindControllers 	9

5.1.2	u2aGetSerialNumber	9

5.1.3	u2aOpen	10

5.1.4	u2aClose	10

5.1.5	u2aReadResponse (Deprecated)	11

5.1.6	u2aSetReceiveTimeout	11

5.1.7	u2aEnableDeviceDetect	12

5.1.8	u2aEnableDebugLogging	13

5.1.9	u2aSuppressSplash	14

5.1.10	u2aSetAsyncIOCallback	15

5.2	I2C Functions	16

5.2.1	u2aI2C_Control	16

5.2.2	u2aI2C_RegisterRead	16

5.2.3	u2aI2C_RegisterWrite	17

5.2.4	u2aI2C_MultiRegisterRead	17

5.2.5	u2aI2C_MultiRegisterWrite	18

5.2.6	u2aI2C_InternalRead	18

5.2.7	u2aI2C_InternalWrite	19

5.2.8	u2aI2C_RawRead	20

5.2.9	u2aI2C_RawWrite	20

5.2.10	u2aI2C_BlockWriteBlockRead	21

5.3	SPI Functions	22

5.3.1	u2aSPI_Control	22

5.3.2	u2aSPI_WriteAndRead	24

5.3.3	u2aSPI_WriteAndReadEx	24

5.4	SMBus Functions	25

5.4.1	u2aSMBUS_Control	25

5.4.2	u2aSMBUS_SendByte	26

5.4.3	u2aSMBUS_WriteByte	26

5.4.4 u2aSMBUS_WriteWord	27

5.4.5	u2aSMBUS_WriteBlock	28

5.4.6	u2aSMBUS_ReceiveByte	29

5.4.7	u2aSMBUS_ReadByte	29

5.4.8 u2aSMBUS_ReadWord	30

5.4.9	u2aSMBUS_ReadBlock	31

5.4.10	u2aSMBUS_BlockWriteBlockRead	32

5.4.11	u2aSMBUS_GetEchoBuffer	32

5.5	RFFE Functions	33

5.5.1	u2aRFFE_RegisterZeroWrite	33

2

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.5.2	u2aRFFE_RegisterWrite	33

5.5.3	u2aRFFE_ExtRegisterWrite	34

5.5.4	u2aRFFE_ExtRegisterWriteLong	34

5.5.5	u2aRFFE_RegisterRead	35

5.5.6	u2aRFFE_ExtRegisterRead	35

5.5.7	u2aRFFE_ExtRegisterReadLong	36

5.6	ADC Functions	37

5.6.1	u2aADC_Control	37

5.6.2	u2aADC_ConvertAndRead	38

5.6.3	u2aADC_Enable	39

5.6.4	u2aADC_SetReference	39

5.6.5	u2aADC_Acquire	40

5.6.6	u2aADC_AcquireTriggered	41

5.6.7	u2aADC_GetData	41

5.6.8	u2aADC_GetStatus	43

5.7	DAC Functions	45

5.7.1	u2aDACs_Write	45

5.7.2	u2aDAC_SetValue	45

5.8	PWM Functions	47

5.8.1	u2aPWM_Control	47

5.9	UART Functions	50

5.9.1	u2aUART_Control	50

5.9.2	u2aUART_Write	51

5.9.3	u2aUART_Read	51

5.9.4	u2aUART_DisableReceiver	51

5.9.5	u2aUART_GetRxCount	52

5.9.6 u2aUART_SetMode	52

5.10	GPIO Functions	53

5.10.1	u2aGPIO_WriteControl	53

5.10.2	u2aGPIO_WriteState	54

5.10.3	u2aGPIO_ReadState	55

5.10.4	u2aGPIO_SetPort	55

5.10.5	u2aGPIO_WritePort	56

5.10.6	u2aGPIO_ReadPort	57

5.10.7	u2aGPIO_WritePulse	57

5.11	Memory Functions	59

5.11.1	u2aMSP430_ByteRead	59

5.11.2	u2aMSP430_ByteWrite	59

5.11.3	u2aMSP430_WordRead	59

5.11.4	u2aMSP430_WordWrite	60

5.11.5	u2aMSP430_MemoryRead	60

5.11.6	u2aMSP430_MemoryWrite	61

5.12	FEC Functions	62

5.12.1	u2aFEC_Control (Deprecated)	62

5.12.2	u2aFEC_Configure	63

5.12.3	u2aFEC_CountAndRead	64

5.12.4	u2aFEC_PurgeBuffer	64

5.12.5	u2aFEC_GetResult	65

5.13	Interrupt Functions	66

5.13.1	u2aInterrupt_Control	66

5.13.2	u2aInterrupt_CheckReceived	66

5.13.3	u2aInterrupt_SetHandler	67

5.14	Digital Capture Functions	69

5.14.1	u2aDigital_Capture	69

5.15	EasyScaleTM Functions	71

3

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.15.1	u2aEasyScale_Control	71

5.15.2	u2aEasyScale_Write	72

5.15.3	u2aEasyScale_Read	73

5.15.4	u2aEasyScale_WriteAndRead	74

5.16	DisplayScaleTM Functions	75

5.16.1	u2aDisplayScale_Setup	75

5.16.2	u2aDisplayScale_WriteReg	75

5.16.3	u2aDisplayScale_ReadReg	76

5.16.4	u2aDisplayScale_WriteAndRead	77

5.17	OneWire Functions	78

5.17.1	u2aOneWire_SetMode	78

5.17.2	u2aOneWire_PulseSetup	78

5.17.3	u2aOneWire_PulseWrite	80

5.17.4	u2aOneWire_PulseWriteEx	80

5.17.5	u2aOneWire_SetOutput	81

5.18	Power Functions	82

5.18.1	u2aPower_WriteControl (Deprecated)	82

5.18.2	u2aPower_Enable	82

5.18.3	u2aPower_SetVoltageRef	83

5.18.4	u2aPower_ReadStatus	83

5.19	Miscellaneous Functions	85

5.19.1	u2aFirmwareVersion_Read	85

5.19.2	u2aLED_WriteControl (USB2ANY)	85

5.19.3	u2aLED_SetState (OneDemo)	86

5.19.4	u2aClock_Control	87

5.20	Status Functions	88

5.20.1	u2aStatus_IsUSB2ANYConnected	88

5.20.2	u2aStatus_GetErrorCode	88

5.20.3	u2aStatus_GetText	89

5.20.4	u2aStatus_GetControllerType	89

5.20.5	u2aStatus_EVMDetect	90

5.20.6	u2aStatus_GetBoardRevision	91

Appendix A: Error Codes	92

Appendix B: Exported Symbols	94

Appendix C: Visual Basic Interface (VB6)	96

Appendix D: SMBus Details	99

Appendix E: USB2ANY Schematic	100

Appendix F: USB2ANY Cable Connections	103

4

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Preface

Read This First

About This Manual

This user's guide describes the functions and operation of the USB2ANY Interface Adapter, from different aspects of hardware design, firmware programming, communication protocols, GUI and PC libraries, etc.

How to Use This Manual

This document contains the following chapters:

- Preface

- Chapter 1 – Introduction

- Chapter 2 – Overview of Hardware Design

- Chapter 3 – Programming and Communications Protocol

- Chapter 4 – API Summary

- Chapter 5 – API Function Reference

- Appendices

Information about Cautions and Warnings

FCC Warning

5

This equipment is intended for use in a laboratory test environment only. It generates, uses, and can radiate radio frequency energy and has not been tested for compliance with the limits of computing devices pursuant to subpart J of part 15 of FCC rules, which are designed to provide reasonable protection against radio frequency interference. Operation of this equipment in other environments may cause interference with radio communications, in which case, the user, at his/her own expense, will be required to take whatever measures may be required to correct this interference.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

1 Introduction

This document describes the API library (USB2ANY.DLL), which provides a set of functions to simplify the interfacing and operation of the USB2ANY controller board.

The USB2ANY Controller Board is a small dongle that, via a USB connection to a computer, enables access to devices using the following interfaces:

- I2C

- SPI

- ADC

- DAC

- PWM

- UART

- GPIO

- Shared memory

- FEC

- Interrupts

- EasyScaleTM

The USB2ANY board can source regulated 3.3v and 5.0v DC power to the target device, up to a maximum of 300ma (combined 3.3v and 5.0v).

The API library is intended for use on computers running the Windows operating system (XP through Windows 7) and can be accessed by any language (e.g., Visual C++, Visual Basic, Python, etc.) that is capable of interfacing with a DLL (dynamic-linked library). The DLL can be linked manually, using Windows API functions, or automatically at run-time. An object library file (USB2ANY.LIB) is provided to simplify interfacing.

2 Overview of Hardware Design

See Appendix D: on page 99 for drawings.

TBD

3 Programming and Communications Protocol

See Appendix B: Exported Symbols on page 94 for the names required for linkage to the API functions.

See Appendix C: Visual Basic Interface (VB6) on page 96 for VB6 declarations of USB2ANY.DLL subroutines and functions.

TBD

6

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

4 API Summary

This section provides a quick summary of the API functions available in the USB2ANY.DLL library. The list is organized by function type, in order to expedite finding the desired function. The functions are listed by function name a brief one-line description for each function.

This list is intended only as a Quick Reference and assumes that the reader has experience and/or knowledge of the API. See section 5 for detailed descriptions of the functions.

| Controller Functions |  |
| --- | --- |
| u2aFindControllers | Scans the USB bus, enumerating USB2ANY devices and creating a list of the devices it finds. The list can be read using the u2aGetSerialNumber function. |
| u2aGetSerialNumber | Returns the serial number of one of the USB2ANY devices found by a previous call to the u2aFindControllers function. |
| u2aOpen | Opens communication with the USB2ANY controller that has the specified serial number. |
| u2aClose | Closes communication with the USB2ANY controller associated with the specified handle. |
| u2aReadResponse | Reads the response from any of the other functions that return data. |
| I2C Functions |  |
| I2C_Control | Sets the communications parameters (speed, address length, and pullup state) for the I2C interface. |
| u2aI2C_RegisterRead | Reads a single byte from a register of a device on the I2C bus. |
| u2aI2C_RegisterWrite | Writes a single byte to a register of a device on the I2C bus. |
| u2aI2C_MultiRegisterRead | Reads data from multiple registers of a device on the I2C bus. |
| u2aI2C_MultiRegisterWrite | Writes data to multiple registers of a device on the I2C bus. |
| u2aI2C_InternalRead | Reads data from a device on the I2C bus, using an internal address. |
| u2aI2C_InternalWrite | Writes data to an internal address of a device on the I2C bus. |
| u2aI2C_RawRead | Reads raw data from a device on the I2C bus. |
| u2aI2C_RawWrite | Writes raw data to a device on the I2C bus. |
| SPI Functions |  |
| u2aSPI_Control | Sets the parameters for SPI transactions. |
| u2aSPI_WriteAndRead | Simultaneously writes and reads data from an SPI device. |
| ADC, DAC, and PWM Functions |  |
| u2aADC_Control | Sets the parameters for the ADC pins. |
| u2aADC_ConvertAndRead | Triggers an ADC conversion and reads the results. |
| u2aDACs_Write | Writes data to the specified DAC. |
| u2aPWM_Control | Sets the parameters for the PWM pins. |

7

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

8

| UART Functions |  |
| --- | --- |
| u2aUART_Control | Sets the USART parameters for serial communication. |
| u2aUART_Write | Writes serial data via the USART. |
| GPIO Functions |  |
| u2aGPIO_WriteControl | Writes control data to all of the GPIO pins, simultaneously. |
| u2aGPIO_WriteState | Sets the output state of all GPIO pins, simultaneously. |
| u2aGPIO_ReadState | Reads the input state of all of the GPIO pins, simultaneously. |
| u2aGPIO_SetPort | Configures a single GPIO pin as an output or input (with resistor options). |
| u2aGPIO_WritePort | Sets the state of a single GPIO output pin. |
| u2aGPIO_ReadPort | Reads the state of a single GPIO input pin. |
| Memory Functions |  |
| u2aMSP430_ByteRead | Reads a single byte from the MSP430 memory. |
| u2aMSP430_ByteWrite | Writes a single byte to the MSP430 memory. |
| u2aMSP430_WordRead | Reads a single word from the MSP430 memory. |
| u2aMSP430_WordWrite | Writes a single word to the MSP430 memory. |
| Miscellaneous Functions |  |
| u2aFirmwareVersion_Read | Reads the firmware version number of the USB2ANY controller. |
| u2aPower_WriteControl | Enables/disables the 3.3V and 5.0V power outputs. |
| u2aPower_ReadStatus | Reads the status of the 3.3V and 5.0V power outputs. |
| u2aLED_WriteControl | Used to control the LED. |
| u2aClock_Control | Sets the clock divider. |

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5 API Function Reference

5.1	Controller Functions

5.1.1 u2aFindControllers

int u2aFindControllers(void)

int CU2AClass:: FindControllers()

This function scans the USB bus, enumerating USB2ANY devices and creating a list of the devices it finds. The list can be read using the u2aGetSerialNumber function.

Parameters:
None

Return:

Returns the number of USB2ANY devices found or zero, if no devices were found.

5.1.2 u2aGetSerialNumber

int u2aGetSerialNumber(int index, char *SerialNumber)
int CU2AClass:: GetSerialNumber(int index, char *SerialNumber)

This function returns the serial number of one of the USB2ANY devices found by a previous call to the u2aFindControllers function.

Parameters:

| index | An index into the list of USB2ANY devices that was returned by the u2aFindControllers function. This must be a number in the range of zero to n-1 where n is the number returned by the u2aFindControllers function. To get the serial number of the first device, set index to zero. To enumerate remaining devices, set index to -1. This may also be set to any valid index to get the serial number associated with that index. An error is returned if index is out of range. |
| --- | --- |

9

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

| SerialNumber | A char buffer to receive the selected serial number string. The buffer should have room for at least 40 characters (defined as SERNUM_LEN in USB2ANY_SDK.H). |
| --- | --- |

Return:

Returns zero on success. If index is out of range, or there are no more devices to enumerate,

ERR_PARAM_OUT_OF_RANGE is returned.

5.1.3 u2aOpen

HANDLE u2aOpen (char *SerialNumber)

HANDLE CU2AClass:: Open (char *SerialNumber)

This function opens communication with the USB2ANY controller that has the specified serial number.

Parameters:

| SerialNumber | A serial number string that was returned by a call to the u2aGetSerialNumber function. |
| --- | --- |

Comments:

This function supports an “Easy Open” method that may be utilized by simple applications that use only one USB2ANY device connected to the computer. To use this method, the SerialNumber parameter is set to an empty string (“”) or NULL (0). This will cause the u2aOpen function to look for the first available USB2ANY, get its serial number, and then attempt to open it.

Return:

Returns a handle, which must be used for subsequent calls to API functions. The handle is always

a positive number (never zero). If an error occurs, a negative error code is returned.

5.1.4 u2aClose

int u2aClose (U2A_HANDLE handle)
int CU2AClass:: Close ()

This function closes communication with the USB2ANY controller associated with the specified handle.

10

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.1.5 u2aReadResponse (Deprecated)

int u2aReadResponse(U2A_HANDLE handle, BYTE *pBuffer, DWORD dwBufferSize) This function reads the response from any of the other functions that return data.

Warning: This function is deprecated. The function is not necessary and performs no useful function in the USB2ANY API V2.6.0.0 and later.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| pBuffer | Pointer to a buffer, which will receive the data. Care must be taken to ensure that the buffer is large enough to hold the expected data. To be safe, you can make the size of the buffer MAXIMUM_DATA_SIZE (defined in USB2ANY_SDK.H) bytes. |
| dwBufferSize | The size of the buffer (in bytes), pointed to by pBuffer. |

Return:

Always returns ERR_UNIMPLEMENTED_FUNCTION when called in API V2.6.0.0, or later.

5.1.6 u2aSetReceiveTimeout

int u2aSetReceiveTimeout (int milliseconds)

int CU2AClass:: SetReceiveTimeout (int milliseconds)

This function sets the timeout value, in milliseconds, used during USB receive operations.

Parameters:

| milliseconds | Timeout value, in milliseconds (minimum 2, maximum 200) |
| --- | --- |

11

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Comments:

The default receive timeout is 25 milliseconds. This value should be adequate for virtually all of the API functions. However, the values of certain parameters used when calling some API functions may require longer timeouts. For example, using slower baud rates (e.g. 300 baud) with the UART functions may cause timeouts to occur when sending more than a few bytes of data.

You can temporarily increase the timeout while calling a single API function (or a group of functions) as follows:

old_value = u2aSetReceiveTimeout(new_value);	// set new value, saving the old value

u2aUART_Write(handle, 10, data);	// call the API function

u2aSetReceiveTimeout(old_value);	// restore the saved timeout value

Note that leaving the receive timeout set to larger values may adversely impact the overall performance of other API functions.

Return:

Returns the previous timeout value on success. If an error occurs, the timeout value is not

changed and one of the following error codes is returned:

- ERR_PARAM_OUT_OF_RANGE

- ERR_COM_PORT_NOT_OPEN

- ERR_OPERATION_FAILED

5.1.7 u2aEnableDeviceDetect

BOOL u2aEnableDeviceDetect (U2A_HANDLE handle, BOOL enable, void *Callback) BOOL CU2AClass:: EnableDeviceDetect (BOOL enable, void *Callback)

This function enables/disables the detection and automatic reporting of the controller device (USB2ANY or OneDemo) plug and unplug events.

Parameters:

| handle | Valid handle (obtained from U2A_Open) for the USB2ANY device. |
| --- | --- |
| enable | Determines whether to detect and report changes of the device connection status. Disable = 0 Enable = 1 |
| Callback | The address of the callback function to be called when a controller board is connected or disconnected. If set to NULL, no function is called. Note that board connection/disconnection status can be determined “manually” by calling the u2aStatus_IsUSB2ANYConnected() |
|  | function, regardless of whether the u2aEnableDeviceDetect() function has been called. |

12

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Comments:

If Callback is set to a non-Null value, the function at the specified address will be called whenever the controller board is connected or disconnected. The callback function must have the following prototype:

void __stdcall Callback(BOOL bConnected, const char *szSerialNumber)

There will be two parameters passed to the function. The first parameter is a Boolean value that denotes the connection status and will be zero (0) if a board was disconnected or one (1) if a board was connected. The second parameter is a pointer to char string containing the serial number of the board that was connected or disconnected.

If the use of a callback function is not desired, this function should not be used. Instead, use the u2aStatus_IsUSB2ANYConnected() function.

Return:

Returns a Boolean value – TRUE on success, FALSE on failure.

5.1.8 u2aEnableDebugLogging

int u2aEnableDebugLogging (BOOL enable)

int CU2AClass:: EnableDebugLogging (BOOL enable)

This function enables/disables the logging of debug messages for every API function that is called. The debug log is intended to help with development and debugging of USB2ANY and OneDemo applications that use the API functions provided by the SDK.

Parameters:

| enable | Determines whether to log information for each function called. Disable = 0 Enable = 1 |
| --- | --- |

Comments:

Logging of debug information is enabled when enable is set to a TRUE (i.e., non-zero) value. The log file shows the full path and version number of the currently loaded USB2ANY.DLL library. The debug information logged for each function call consists of a time/date stamp, the function name, any parameters passed to the function (with values), and the returned value. Additional information may also be logged.

The debug log files are typically located in the USB2ANY\Logs sub-folder of the Windows Documents folder (e.g., C:\Users\YourUserName\Documents\USB2ANY\Logs). The name of the debug log file will be <AppName> API Debug.log, where <AppName> is the name of the

13

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

application program that loaded USB2ANY.DLL. For example, running the USB2ANY Explorer.exe program will create a debug log file named USB2ANY Explorer API Debug.log. The contents of the debug log file will look something like this:

2013-08-26 12:04:51.692 llllllllllllllllllllllllllllllllllllllllllllllll SESSION START llllllllllllllllllllllllllllllllllllllllllllllllllll

2013-08-26 12:04:51.692 Loading C:\Windows\SysWOW64\USB2ANY.dll (v2.6.1.1)

2013-08-26 12:04:51.896

2013-08-26 12:04:51.897 u2aEnableAPIProfiling(enable=1) [3279, TI_USB2ANY.cpp]

2013-08-26 12:04:51.899 Returning value 0 [3285, TI_USB2ANY.cpp]

2013-08-26 12:04:51.922

2013-08-26 12:04:51.925 u2aEnableDeviceDetect(enable=1, pfnCallback=0x002E6680) [480, TI_USB2ANY.cpp]

2013-08-26 12:05:00.832

2013-08-26 12:05:00.834 u2aFindControllers() [813, TI_USB2ANY.cpp]

2013-08-26 12:05:00.917 Found controller S/N AF14904609001C00 [799, TI_USB2ANY.cpp]

2013-08-26 12:05:00.919 Returning value 1 [808, TI_USB2ANY.cpp]

2013-08-26 12:05:00.920 Elapsed time: 89.77163 ms.

2013-08-26 12:05:00.921 Returning value 1 [819, TI_USB2ANY.cpp]

2013-08-26 12:05:00.923 Return code set to 1 [493, TI_USB2ANY.cpp]

2013-08-26 12:05:00.925 Elapsed time: 94.56611 ms.

2013-08-26 12:05:00.926 Returning value 1 [498, TI_USB2ANY.cpp]

Return:

The u2aEnableDebugLogging function returns the previous enable state of debug logging (TRUE if it was enabled before executing this function, otherwise FALSE). If an error occurs, a negative error code is returned.

5.1.9 u2aSuppressSplash

BOOL u2aSuppressSplash (BOOL suppress)

BOOL CU2AClass:: SuppressSplash (BOOL suppress)

This function enables/disables the display of the splash screen, which occurs the first time that the u2aOpen function is called.

Parameters:

| suppress | Determines whether the splash screen is displayed. Set TRUE to suppress the splash screen or FALSE to allow it to be displayed. |
| --- | --- |

Comments:

The splash screen is displayed only once per session, when the u2aOpen function is called. In

order for this function to have an effect, it must be called before the first call to u2aOpen.

Return:

The u2aSuppressSplash function returns the previous splash suppression flag (TRUE if suppression was enabled before executing this function, otherwise FALSE). If an error occurs, a negative error code is returned.

14

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.1.10 u2aSetAsyncIOCallback

int u2aSetAsyncIOCallback (U2A_HANDLE handle, void *Callback, int nFunctionID) int CU2AClass:: SetAsyncIOCallback (void *Callback, int nFunctionID)

This function sets the address of the user function to be called when the specified function’s I/O completes.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| Callback | The address of the callback function to be called when the I/O operation completes. If set to NULL, no function is called. See Comments section for more information. |
| nFunctionID | The ID of the function for which the callback will be set. Valid values are: Cmd_I2C_ReadInternal = 101 (0x65) Cmd_I2C_WriteInternal = 102 (0x66) |

Comments:

The u2aSetAsyncIOCallback function allows certain I/O functions to have their results returned asynchronously. Identifiers for the API functions that allow asynchronous operation are shown in the description of the nFunctionID parameter. Note that the callback will remain in effect until cancelled, which is done by calling this function with the Callback parameter set to NULL.

When Callback is set to a non-null value, a function at the specified address will be called when I/O for the specified function completes. The callback function must have the following prototype:

void _stdcall CallBack(int result, BYTE *data)

There will be two parameters passed to the callback function. The first parameter, result, will be set to either the number of bytes transferred (zero or positive) or an error code (always negative).

The second parameter, data, is a pointer to the returned data on a successful read operation. The data parameter is always set to NULL on write operations, as there is no data returned.

To disable the callback, call this function with the Callback parameter set to NULL. In this case, the nFunctionID parameter must also be set to the appropriate function ID.

Return:

15

u2aSetAsyncIOCallback returns zero on success, or a negative error code if an error occurs.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.2 I2C Functions

5.2.1 u2aI2C_Control

int u2aI2C_Control (U2A_HANDLE handle, I2C_Speed Speed,

I2C_AddressLength AddressLength, I2C_PullUps PullUps)

int CU2AClass:: I2C_Control (I2C_Speed Speed, I2C_AddressLength AddressLength, I2C_PullUps PullUps)

This function sets the communications parameters (speed, address length, and pullup state) for the I2C interface.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |  |  |
| --- | --- | --- | --- |
| Speed | The bitrate for I2C communications. Valid values are: |  |  |
|  | I2C_100kHz | = | 0 |
|  | I2C_400kHz | = | 1 |
|  | I2C_10kHz | = | 2 |
| AddressLength | Size of the I2C slave device address. May be 7 or 10 bits. Valid values are: |  |  |
|  | I2C_7Bits | = | 0 |
|  | I2C_10Bits | = | 1 |
| PullUps | Sets the state of the I2C pullups. May be 0 (off) or 1 (on). Valid values are: |  |  |
|  | I2C_PullUps_OFF | = | 0 |
|  | I2C_PullUps_ON | = | 1 |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.2.2 u2aI2C_RegisterRead

int u2aI2C_RegisterRead (U2A_HANDLE handle, UInt16 I2C_Address, Byte RegisterAddress)

16

int CU2AClass:: I2C_RegisterRead (UInt16 I2C_Address, Byte RegisterAddress) This function reads a single byte from a register of a device on the I2C bus.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| I2C_Address | The address of the I2C device. May be 7 or 10 bits. See u2aI2C_Control. |
| RegisterAddress | Address of the register in the I2C slave device. Must be a single byte value. |

Return:

Returns the value of the byte read on success. If an error occurs, a negative error code is returned.

5.2.3 u2aI2C_RegisterWrite

int u2aI2C_RegisterWrite(U2A_HANDLE handle, UInt16 I2C_Address,
Byte RegisterAddress, Byte Value)

int CU2AClass:: I2C_RegisterWrite(UInt16 I2C_Address, Byte RegisterAddress, Byte Value)

This function writes a single byte to a register of a device on the I2C bus.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| I2C_Address | The address of the I2C device. May be 7 or 10 bits. See u2aI2C_Control. |
| RegisterAddress | Address of the register in the I2C slave device. Must be a single byte value. |
| Value | The single-byte value to be written to the specified register. |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.2.4 u2aI2C_MultiRegisterRead

int u2aI2C_MultiRegisterRead (U2A_HANDLE handle, UInt16 I2C_Address,

Byte StartingRegisterAddress, Byte nBytes, Byte *Data)

int CU2AClass:: I2C_MultiRegisterRead (UInt16 I2C_Address,

Byte StartingRegisterAddress, Byte nBytes,

Byte *Data)

17

This function reads data from multiple registers of a device on the I2C bus.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| I2C_Address | The address of the I2C device. May be 7 or 10 bits. See u2aI2C_Control. |
| StartingRegisterAddress | Address of the first register to read. Must be a single byte value. |
| nBytes | The number of bytes (i.e., registers) to be read. |
| Data | Pointer to array of bytes to receive the data read. |

Return:

Returns number of bytes read on success. If an error occurs, a negative error code is returned.

5.2.5 u2aI2C_MultiRegisterWrite

int u2aI2C_MultiRegisterWrite (U2A_HANDLE handle, UInt16 I2C_Address,

Byte StartingRegisterAddress, Byte nBytes, Byte *Data)

int CU2AClass::I2C_MultiRegisterWrite (UInt16 I2C_Address,

Byte StartingRegisterAddress, Byte nBytes,

Byte *Data)

This function writes data to multiple registers of a device on the I2C bus.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| I2C_Address | The address of the I2C device. May be 7 or 10 bits. See u2aI2C_Control. |
| StartingRegisterAddress | Address of the first register to write. Must be a single byte value. |
| nBytes | The number of bytes to be written. |
| Data | Pointer to array of bytes to be written. |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.2.6 u2aI2C_InternalRead

int u2aI2C_InternalRead (U2A_HANDLE nHandle, UInt16 I2C_Slave_Address, UInt16 InternalAddress, Byte IntAddrSize, UInt16 nBytes, Byte *Data)

int CU2AClass::I2C_InternalRead (UInt16 I2C_Slave_Address,

UInt16 InternalAddress, Byte IntAddrSize, UInt16 nBytes,

Byte *Data)

18

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

This function reads data from a device on the I2C bus, using an internal address.

Parameters:

| nHandle | Valid handle (obtained from u2aOpen) for the USB2ANY device. |
| --- | --- |
| I2C_Slave_Address | The address of the I2C device. May be 7 or 10 bits. See u2aI2C_Control. |
| InternalAddress | Internal address of the data to read. |
| IntAddrSize | Number of internal address bytes. Must be 0, 1, or 2. |
| nBytes | The number of bytes to be read. |
| Data | Pointer to array of bytes to receive the data read. |

Return:

If a positive number, the number of bytes actually read. On failure, an error code is returned (always negative).

5.2.7 u2aI2C_InternalWrite

int u2aI2C_InternalWrite (U2A_HANDLE nHandle, UInt16 I2C_Slave_Address, UInt16 InternalAddress, Byte IntAddrSize, UInt16 nBytes, Byte *Data)

int CU2AClass::I2C_InternalWrite (UInt16 I2C_Slave_Address,

UInt16 InternalAddress, Byte IntAddrSize, UInt16 nBytes,

Byte *Data)

This function writes data to an internal address of a device on the I2C bus.

Parameters:

| nHandle | Valid handle (obtained from u2aOpen) for the USB2ANY device. |
| --- | --- |
| I2C_Slave_Address | The address of the I2C device. May be 7 or 10 bits. See u2aI2C_Control. |
| InternalAddress | Internal target address for the data to be written. |
| IntAddrSize | Number of internal address bytes. Must be 0, 1, or 2. |
| nBytes | The number of bytes to be written. |
| Data | Pointer to array of bytes to be written. |

Return:

19

If a positive number, the number of bytes actually written. On failure, an error code is returned (always negative).

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.2.8 u2aI2C_RawRead

int u2aI2C_RawRead (U2A_HANDLE handle, UInt16 I2C_Address, Byte nBytes, Byte *Data)

int CU2AClass::I2C_RawRead (UInt16 I2C_Address, Byte nBytes, Byte *Data) This function reads raw data from a device on the I2C bus.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| I2C_Address | The address of the I2C device. May be 7 or 10 bits. See u2aI2C_Control. |
| nBytes | The number of bytes to be read. |
| Data | Pointer to a buffer to receive the data read. |

Return:

On success, returns number of bytes read. If an error occurs, a negative error code is returned.

5.2.9 u2aI2C_RawWrite

int u2aI2C_RawWrite (U2A_HANDLE handle, UInt16 I2C_Address, Byte nBytes, Byte *Data)

int CU2AClass::I2C_RawWrite (UInt16 I2C_Address, Byte nBytes, Byte *Data) This function writes raw data to a device on the I2C bus.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| I2C_Address | The address of the I2C device. May be 7 or 10 bits. See u2aI2C_Control. |
| nBytes | The number of bytes to be written. |
| Data | Pointer to array of bytes to be written. |

Return:

20

Returns zero on success. If an error occurs, a negative error code is returned.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.2.10 u2aI2C_BlockWriteBlockRead

int u2aI2C_BlockWriteBlockRead (U2A_HANDLE handle, UInt16 I2C_Address, Byte nWriteBytes, Byte *WriteData, Byte nReadBytes, Byte *ReadData)

int CU2AClass:: u2aI2C_BlockWriteBlockRead (UInt16 I2C_Address,

Byte nWriteBytes, Byte *WriteData, Byte nReadBytes,

Byte *ReadData)

This function writes a specified number of data bytes to a device on the I2C bus, then, after a repeated START (i.e., no STOP), reads a specified number of data bytes from the device.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| I2C_Address | The address of the I2C device. May be 7 or 10 bits. See u2aI2C_Control. |
| nWriteBytes | The number of bytes to be written. |
| WriteData | Pointer to array of bytes to be written. |
| nReadBytes | The number of bytes to be read. |
| ReadData | Pointer to a buffer to receive the data read. |

Return:

21

Returns zero on success. If an error occurs, a negative error code is returned.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.3	SPI Functions

5.3.1 u2aSPI_Control

int u2aSPI_Control (U2A_HANDLE handle, SPI_ClockPhase SPI_ClockPhase,

SPI_ClockPolarity SPI_ClockPolarity,
SPI_BitDirection SPI_BitDirection,
SPI_CharacterLength SPI_CharacterLength,
SPI_ChipSelectType SPI_CSType,
SPI_ChipSelectPolarity SPI_CSPolarity,
Byte DividerHigh, Byte DividerLow)

int CU2AClass::SPI_Control (SPI_ClockPhase SPI_ClockPhase,

SPI_ClockPolarity SPI_ClockPolarity,
SPI_BitDirection SPI_BitDirection,
SPI_CharacterLength SPI_CharacterLength,
SPI_ChipSelectType SPI_CSType,
SPI_ChipSelectPolarity SPI_CSPolarity,
Byte DividerHigh, Byte DividerLow)

This function sets the parameters for SPI transactions.

Parameters:

| handle |  | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- | --- |
| SPI_ClockPhase |  | Defines the edge of the clock on which data is captured: SPI_Capture_On_Trailing_Edge = 0 SPI_Capture_On_Leading_Edge = 1 |
| SPI_ClockPolarity |  | Defines which clock state is considered inactive: SPI_Inactive_State_Low = 0 SPI_Inactive_State_High = 1 |
| SPI_BitDirection |  | Defines which bit of each byte is sent first: SPI_LSB_First = 0 SPI_MSB_First = 1 |
| SPI_CharacterLength |  | Defines the number of data bits in each byte transferred: SPI__8_Bit = 0 SPI__7_Bit = 1 |
| SPI_CSType |  | Defines how the CS (chip select) signal is generated: SPI_ With_Every_Byte = 0 SPI_With_Every_Packet = 1 SPI_With_Every_Word = 2 SPI_No_CS = 3 SPI_Packets_With_MOSI_High = 4 SPI_Pulse_After_Packet = 255 |
| SPI_CSPolarity |  | Defines the state in which the CS (chip select) signal is active: SPI_Active_High = 0 SPI_Active_Low = 1 |
| DividerHigh |  | The high byte of the clock divider that sets the SPI bit rate. |

22

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

| DividerLow | The low byte of the clock divider that sets the SPI bit rate. |
| --- | --- |

Comments:

The bit rate divisor can be calculated as follows:

divider = 24000000 / bit_rate_bps
_DividerHigh = (divider >> 8) & 0xFF
DividerLow = divider & 0xFF

The table below shows divider constants for common bit rates:

23

Timing with various clock phase and polarity settings

| Bit Rate (kbps) | Divider | _DividerHigh | _DividerLow |
| --- | --- | --- | --- |
| 10 | 2400 | 9 | 96 |
| 25 | 960 | 3 | 192 |
| 50 | 480 | 1 | 224 |
| 100 | 240 | 0 | 240 |
| 125 | 192 | 0 | 192 |
| 200 | 120 | 0 | 120 |
| 250 | 96 | 0 | 96 |
| 400 | 60 | 0 | 60 |
| 500 | 48 | 0 | 48 |
| 800 | 30 | 0 | 30 |
| 1000 | 24 | 0 | 24 |
| 2000 | 12 | 0 | 12 |
| 4000 | 6 | 0 | 6 |
| 8000 | 3 | 0 | 3 |

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

When is SPI_CSType is set to SPI_Packets_With_MOSI_High (type 4), the chip select behavior is the same as when SPI_CSType is set to SPI_With_Every_Packet (type 1), but the MOSI signal will be held high between packets.

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.3.2 u2aSPI_WriteAndRead

int u2aSPI_WriteAndRead (U2A_HANDLE handle, Byte nBytes, Byte *Data) int CU2AClass::SPI_WriteAndRead (Byte nBytes, Byte *Data)

This function simultaneously writes and reads data from an SPI device.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| nBytes | The number of bytes to be written/read. |
| Data | Pointer to array of bytes to be written/read. |

Comments:

The Data buffer has a dual purpose and is used for both data to be sent and data received. When u2aSPI_WriteAndRead() is called, the buffer contains the data to be sent. On return, the buffer contains the data received.

Return:

Returns number of bytes read on success. If an error occurs, a negative error code is returned.

5.3.3 u2aSPI_WriteAndReadEx

int u2aSPI_WriteAndReadEx (U2A_HANDLE handle, Byte nSS, Byte nBytes, Byte *Data)

int CU2AClass::SPI_WriteAndReadEx (Byte nCS, Byte nBytes, Byte *Data)

This function simultaneously writes and reads data from an SPI device, using a user-specified Slave Select (SS) signal.

24

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| nSS | The number of the GPIO to use for the SS signal. |
| nBytes | The number of bytes to be written/read. |
| Data | Pointer to array of bytes to be written/read. |

Comments:

The nSS parameter specifies the number of the GPIO to be used for the Slave Select (SS) signal. The specified GPIO must be set as an output before calling this function. This needs to be done only once, unless the GPIO function is subsequently changed to something other than output. For example, to use GPIO8 as the SS signal, you could do the following:

Byte nSS = 8;

u2aGPIO_SetPort(handle, nSS, GPIO_Output);
u2aSPI_WriteAndReadEx(handle, nSS, dataSize, dataBuf);

Note that GPIO2, GPIO4, and GPIO5 are used for the SPI interface signals and, therefore, cannot

be used for the SS signal. Also, the default SS signal (used by the u2aSPI_WriteAndRead function) uses GPIO6.

The Data buffer has a dual purpose and is used for both data to be sent and data received. When u2aSPI_WriteAndRead() is called, the buffer contains the data to be sent. On return, the buffer contains the data received.

Return:

Returns number of bytes read on success. If an error occurs, a negative error code is returned.

5.4	SMBus Functions

5.4.1	u2aSMBUS_Control

int u2aSMBUS_Control (U2A_HANDLE handle, SMBUS_FLAGS Flags) int CU2AClass::SMBUS_Control (SMBUS_FLAGS Flags)

25

This function sets the default value to be used when SM BUS_PEC_DEFAULT is used as the Flags parameter for other SMBus read/write functions.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| Flags | Must be one of the following flag values: SMBUS_PEC_OFF = 1 SMBUS_PEC_CRC8 = 2 |

Return:

Returns zero on success. On failure, an error code is returned (always negative).

5.4.2	u2aSMBUS_SendByte

int u2aSMBUS_SendByte (U2A_HANDLE handle, UInt16 I2C_Slave_Address, Byte CommandCode, SMBUS_FLAGS Flags)

int CU2AClass::SMBUS_SendByte (UInt16 I2C_Slave_Address, Byte CommandCode, SMBUS_FLAGS Flags)

This function sends a single byte command to an SMBus device.
Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| I2C_Slave_Address | The address of the SMBus device. Must be 7 bits. |
| CommandCode | The SMBus command to be sent. |
| Flags | Must be one of the following flag values: SMBUS_PEC_DEFAULT = 0 SMBUS_PEC_OFF = 1 SMBUS_PEC_CRC8 = 2 |

Return:

Returns zero on success. On failure, an error code is returned (always negative).

Comments:

If the SMBUS_PEC_CRC8 flag is used, the PEC is automatically calculated and appended to the

end of the SMBus packet.

5.4.3	u2aSMBUS_WriteByte

26

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

int u2aSMBUS_WriteByte (U2A_HANDLE handle, UInt16 I2C_Slave_Address,

Byte CommandCode, Byte Data, SMBUS_FLAGS Flags)

int CU2AClass::SMBUS_WriteByte (UInt16 I2C_Slave_Address,

Byte CommandCode, Byte Data, SMBUS_FLAGS Flags)

This function sends a single byte to an SMBus device.
Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| I2C_Slave_Address | The address of the SMBus device. Must be 7 bits. |
| CommandCode | The SMBus command to be sent. |
| Data | The 8-bit data byte to be sent. |
| Flags | Must be one of the following flag values: SMBUS_PEC_DEFAULT = 0 SMBUS_PEC_OFF = 1 SMBUS_PEC_CRC8 = 2 |

Return:

Returns zero on success. On failure, an error code is returned (always negative).

Comments:

If the SMBUS_PEC_CRC8 flag is used, the PEC is automatically calculated and appended to the

end of the SMBus packet.

5.4.4	u2aSMBUS_WriteWord

int u2aSMBUS_WriteWord (U2A_HANDLE handle, UInt16 I2C_Slave_Address,

Byte CommandCode, UInt16 Data, SMBUS_FLAGS Flags)

int CU2AClass::SMBUS_WriteWord (UInt16 I2C_Slave_Address,

Byte CommandCode, UInt16 Data, SMBUS_FLAGS Flags)

27

This function sends a single byte to an SMBus device.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| I2C_Slave_Address | The address of the SMBus device. Must be 7 bits. |
| CommandCode | The SMBus command to be sent. |
| Data | The 16-bit data word to be sent. |
| Flags | Must be one of the following flag values: SMBUS_PEC_DEFAULT = 0 SMBUS_PEC_OFF = 1 SMBUS_PEC_CRC8 = 2 |

Return:

Returns zero on success. On failure, an error code is returned (always negative).

Comments:

If the SMBUS_PEC_CRC8 flag is used, the PEC is automatically calculated and appended to the

end of the SMBus packet.

5.4.5	u2aSMBUS_WriteBlock

int u2aSMBUS_WriteBlock (U2A_HANDLE handle, UInt16 I2C_Slave_Address, Byte CommandCode, Byte Length, Byte *Data, SMBUS_FLAGS Flags)

int CU2AClass::SMBUS_WriteBlock (UInt16 I2C_Slave_Address,

Byte CommandCode, Byte Length, Byte *Data,

SMBUS_FLAGS Flags)

This function sends a single byte to an SMBus device.
Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| I2C_Slave_Address | The address of the SMBus device. Must be 7 bits. |
| CommandCode | The SMBus command to be sent. |
| Length | The number of bytes of data to be sent. |
| Data | Pointer to array of data bytes to be sent. |
| Flags | Must be one of the following flag values: SMBUS_PEC_DEFAULT = 0 SMBUS_PEC_OFF = 1 SMBUS_PEC_CRC8 = 2 |

Return:

Returns zero on success. On failure, an error code is returned (always negative).

Comments:

If the SMBUS_PEC_CRC8 flag is used, the PEC is automatically calculated and appended to the

end of the SMBus packet.

28

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.4.6	u2aSMBUS_ReceiveByte

int u2aSMBUS_ReceiveByte (U2A_HANDLE handle, UInt16 I2C_Slave_Address, SMBUS_FLAGS Flags)

int CU2AClass::SMBUS_ReceiveByte (UInt16 I2C_Slave_Address, SMBUS_FLAGS Flags)

This function receives a single byte from an SMBus device.
Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| I2C_Slave_Address | The address of the SMBus device. Must be 7 bits. |
| Flags | Must be one of the following flag values: SMBUS_PEC_DEFAULT = 0 SMBUS_PEC_OFF = 1 SMBUS_PEC_CRC8 = 2 |

Return:

Returns the value of the byte read. On failure, an error code is returned (always negative).

Comments:

If the SMBUS_PEC_CRC8 flag is used, the PEC for the received data is automatically calculated and verified. If the calculated PEC does not match the received PEC, the error code ERR_DATA_READ_ERROR is returned.

5.4.7	u2aSMBUS_ReadByte

int u2aSMBUS_ReadByte (U2A_HANDLE handle, UInt16 I2C_Slave_Address, Byte CommandCode, SMBUS_FLAGS Flags)

int CU2AClass::SMBUS_ReadByte (UInt16 I2C_Slave_Address, Byte CommandCode, SMBUS_FLAGS Flags)

29

This function sends the specified command, and then receives a single byte from an SMBus device.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| I2C_Slave_Address | The address of the SMBus device. Must be 7 bits. |
| CommandCode | The SMBus command to be sent. |
| Flags | Must be one of the following flag values: SMBUS_PEC_DEFAULT = 0 SMBUS_PEC_OFF = 1 SMBUS_PEC_CRC8 = 2 |

Return:

Returns the value of the byte read. On failure, an error code is returned (always negative).

Comments:

If the SMBUS_PEC_CRC8 flag is used, the PEC for the received data is automatically calculated and verified. If the calculated PEC does not match the received PEC, the error code ERR_DATA_READ_ERROR is returned.

5.4.8	u2aSMBUS_ReadWord

int u2aSMBUS_ReadWord (U2A_HANDLE handle, UInt16 I2C_Slave_Address, Byte CommandCode, SMBUS_FLAGS Flags)

int CU2AClass::SMBUS_ReadWord (UInt16 I2C_Slave_Address,

Byte CommandCode, SMBUS_FLAGS Flags)

This function sends the specified command, and then receives a 16-bit word from an SMBus device.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| I2C_Slave_Address | The address of the SMBus device. Must be 7 bits. |
| CommandCode | The SMBus command to be sent. |
| Flags | Must be one of the following flag values: SMBUS_PEC_DEFAULT = 0 SMBUS_PEC_OFF = 1 SMBUS_PEC_CRC8 = 2 |

Return:

Returns the 16-bit value of the data read. On failure, an error code is returned (always negative).

Comments:

If the SMBUS_PEC_CRC8 flag is used, the PEC for the received data is automatically calculated and verified. If the calculated PEC does not match the received PEC, the error code ERR_DATA_READ_ERROR is returned.

30

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.4.9	u2aSMBUS_ReadBlock

int u2aSMBUS_ReadBlock (U2A_HANDLE handle, UInt16 I2C_Slave_Address, Byte CommandCode, Byte Length, Byte *Data, SMBUS_FLAGS Flags)

int CU2AClass::SMBUS_ReadBlock (UInt16 I2C_Slave_Address,

Byte CommandCode, Byte Length, Byte *Data,

SMBUS_FLAGS Flags)

This function sends the specified command, and then receives multiple bytes of data from an SMBus device.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| I2C_Slave_Address | The address of the SMBus device. Must be 7 bits. |
| CommandCode | The SMBus command to be sent. |
| Length | The number of bytes of data to be received. |
| Data | Pointer to an array of bytes to store the data received. |
| Flags | Must be one or more of the following flag values: SMBUS_PEC_DEFAULT = 0 SMBUS_PEC_OFF = 1 SMBUS_PEC_CRC8 = 2 SMBUS_VAR_LEN = 8 |

Return:

Returns the number of bytes received on success. On failure, an error code is returned (always

negative).

Comments:

If the SMBUS_PEC_CRC8 flag is used, the PEC for the received data is automatically calculated and verified. If the calculated PEC does not match the received PEC, the error code ERR_DATA_READ_ERROR is returned. When PEC is on, the SMBUS_VAR_LEN flag may be used to allow less than the requested number of bytes to be received.

31

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.4.10 u2aSMBUS_BlockWriteBlockRead

int u2aSMBUS_BlockWriteBlockRead (U2A_HANDLE nHandle, Byte

I2C_Slave_Address, Byte CommandCode, Byte WriteCount, Byte *WriteData, Byte ReadCount, Byte *ReadData, SMBUS_FLAGS Flags)

int CU2AClass::SMBUS_BlockWriteBlockRead (Byte I2C_Slave_Address,

Byte CommandCode, Byte WriteCount, Byte *WriteData,

Byte ReadCount, Byte *ReadData, SMBUS_FLAGS Flags)

This function writes a command code, followed by a number of data bytes, to the slave address of a
SMBus device on the I2C bus. Then, after a repeated start (no STOP), it reads a number of data bytes.

Parameters:

| nHandle | Valid handle (obtained from u2aOpen) for the USB2ANY device. |
| --- | --- |
| I2C_Slave_Address | The address of the SMBus device. Must be 7 bits. |
| CommandCode | The SMBus command to be sent. |
| WriteCount | The number of data bytes to be written. |
| WriteData | Pointer to array of the data bytes to be written. |
| ReadCount | The number of data bytes of data to be received. |
| ReadData | Pointer to an array of bytes to store the data received. |
| Flags | Must be one of the following flag values: SMBUS_PEC_DEFAULT = 0 SMBUS_PEC_OFF = 1 SMBUS_PEC_CRC8 = 2 |

Return:

If a positive number, the number of bytes read. On failure, an error code is returned (always

negative).

5.4.11 u2aSMBUS_GetEchoBuffer

int u2aSMBUS_GetEchoBuffer (U2A_HANDLE handle, Byte nBufferSize, Byte *Buffer) int CU2AClass::SMBUS_GetEchoBuffer (Byte nBufferSize, Byte *Buffer)

This function retrieves echoed data from a previously called SMBUS function.

Note: This function is currently unavailable.

32

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| nHandle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| nBufferSize | The size, in bytes, of the buffer pointed to by Buffer. |
| Buffer | Pointer to a buffer to receive the echoed read/write data. |

Return:

Returns zero on success. On failure, an error code is returned (always negative).

5.5	RFFE Functions

5.5.1	u2aRFFE_RegisterZeroWrite

int u2aRFFE_RegisterZeroWrite (U2A_HANDLE handle, Byte SlaveAddress, Byte Data)

int CU2AClass::RFFE_RegisterZeroWrite (Byte SlaveAddress, Byte Data) This function writes seven bits of a single byte of data to register zero of a device.

Parameters:

| nHandle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| SlaveAddress | Address of the slave device. Valid address is in the range 0x00 to 0x0F. |
| Data | Data byte to be written. Note that bit 7 is not written. |

Return:

Returns zero or a positive number on success. On failure, an error code is returned (always negative).

5.5.2	u2aRFFE_RegisterWrite

int u2aRFFE_RegisterWrite (U2A_HANDLE handle, Byte SlaveAddress, Byte RegisterAddress, Byte Data)

int CU2AClass::RFFE_RegisterWrite (Byte SlaveAddress, Byte RegisterAddress, Byte Data)

33

This function writes a single byte of data to a register of a device.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| nHandle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| SlaveAddress | Address of the slave device. Valid address is in the range 0x00 to 0x0F. |
| RegisterAddress | Address of register to which data will be written. Valid address is in the range 0x00 to 0x0F. |
| Data | Data to be written. |

Return:

Returns zero or a positive number on success. On failure, an error code is returned (always

negative).

5.5.3	u2aRFFE_ExtRegisterWrite

int u2aRFFE_ExtRegisterWrite (U2A_HANDLE handle, Byte SlaveAddress,

Byte RegisterAddress, Byte nLength, Byte *Data)

int CU2AClass::RFFE_ExtRegisterWrite (Byte SlaveAddress, Byte RegisterAddress, Byte nLength, Byte *Data)

This function writes up to 16 bytes of data to registers of a device.

Parameters:

| nHandle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| SlaveAddress | Address of the slave device. Valid address is in the range 0x00 to 0x0F. |
| RegisterAddress | Address of the first register to which data will be written. Valid address is in the range 0x00 to 0xFF. |
| nLength | Length, in bytes, of data to be written. Valid length is 1 to 16 bytes. |
| Data | Pointer to buffer containing data to be written. |

Return:

Returns zero or a positive number on success. On failure, an error code is returned (always

negative).

5.5.4	u2aRFFE_ExtRegisterWriteLong

int u2aRFFE_ExtRegisterWriteLong (U2A_HANDLE handle, Byte SlaveAddress,

UInt16 RegisterAddress, Byte nLength, Byte *Data)

int CU2AClass::RFFE_ExtRegisterWriteLong (Byte SlaveAddress,

UInt16 RegisterAddress, Byte nLength,

Byte *Data)

34

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

This function writes up to 8 bytes of data to registers of a device.
Parameters:

| nHandle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| SlaveAddress | Address of the slave device. Valid address is in the range 0x00 to 0x0F. |
| RegisterAddress | Address of the first register to which data will be written. Valid address is in the range 0x0000 to 0xFFFF. |
| nLength | Length, in bytes, of data to be written. Valid length is 1 to 8 bytes. |
| Data | Pointer to buffer containing data to be written. |

Return:

Returns zero or a positive number on success. On failure, an error code is returned (always

negative).

5.5.5	u2aRFFE_RegisterRead

int u2aRFFE_RegisterRead (U2A_HANDLE handle, Byte SlaveAddress, Byte RegisterAddress)

int CU2AClass::RFFE_RegisterRead (Byte SlaveAddress, Byte RegisterAddress) This function reads a single byte of data from a register of a device.

Parameters:

| nHandle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| SlaveAddress | Address of the slave device. |
| RegisterAddress | Address of register from which data will be read. |

Return:

Returns the value of the unsigned data read (always positive) on success. On failure, an error code

is returned (always negative).

5.5.6	u2aRFFE_ExtRegisterRead

int u2aRFFE_ExtRegisterRead (U2A_HANDLE handle, Byte SlaveAddress,

Byte RegisterAddress, Byte nLength, Byte *Data)

int CU2AClass::RFFE_ExtRegisterRead (Byte SlaveAddress, Byte RegisterAddress, Byte nLength, Byte *Data)

This function reads up to 16 bytes of data from registers of a device.

35

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| nHandle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| SlaveAddress | Address of the slave device. Valid address is in the range 0x00 to 0x0F. |
| RegisterAddress | Address of the first register from which data will be read. Valid address is in the range 0x00 to 0xFF. |
| nLength | Length, in bytes, of data to be read. Valid length is 1 to 16 bytes. |
| Data | Pointer to buffer that will receive the data read. The buffer must be large enough to receive at least nLength bytes. |

Return:

Returns zero or a positive number on success. On failure, an error code is returned (always negative).

5.5.7	u2aRFFE_ExtRegisterReadLong

int u2aRFFE_ExtRegisterReadLong (U2A_HANDLE handle, Byte SlaveAddress,

UInt16 RegisterAddress, Byte nLength, Byte *Data)

int CU2AClass::RFFE_ExtRegisterReadLong (Byte SlaveAddress,

UInt16 RegisterAddress, Byte nLength,

Byte *Data)

This function reads up to 8 bytes of data from registers of a device.

Parameters:

| nHandle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| SlaveAddress | Address of the slave device. Valid address is in the range 0x00 to 0x0F. |
| RegisterAddress | Address of the first register from which data will be read. Valid address is in the range 0x0000 to 0xFFFF. |
| nLength | Length, in bytes, of data to be read. Valid length is 1 to 8 bytes. |
| Data | Pointer to buffer that will receive the data read. The buffer must be large enough to receive at least nLength bytes. |

Return:

36

Returns zero or a positive number on success. On failure, an error code is returned (always negative).

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.6	ADC Functions

5.6.1 u2aADC_Control

int u2aADC_Control (U2A_HANDLE handle, ADC_PinFunction ADC0,

ADC_PinFunction ADC1, ADC_PinFunction ADC2, ADC_PinFunction ADC3, ADC_VREF VREF)

int CU2AClass::ADC_Control (ADC_PinFunction ADC0, ADC_PinFunction ADC1,

ADC_PinFunction ADC2, ADC_PinFunction ADC3, ADC_VREF VREF)

This function sets the parameters for the ADC pins.
Parameters:

| handle |  | A valid handle, obtained by a call to the u2aOpen function. |  |  |
| --- | --- | --- | --- | --- |
| ADC0 |  | Sets the function of the ADC0 pin: |  |  |
|  |  | ADC_No_Change | = | 0 |
|  |  | ADC_Analog_In | = | 1 |
| ADC1 |  | Sets the function of the ADC1 pin: |  |  |
|  |  | ADC_No_Change | = | 0 |
|  |  | ADC_Analog_In | = | 1 |
| ADC2 |  | Sets the function of the ADC2 pin: |  |  |
|  |  | ADC_No_Change | = | 0 |
|  |  | ADC_Analog_In | = | 1 |
| ADC3 |  | Sets the function of the ADC3 pin: |  |  |
|  |  | ADC_No_Change | = | 0 |
|  |  | ADC_Analog_In | = | 1 |
| VREF |  | Sets the reference voltage for the ADC: |  |  |
|  |  | ADC_VREF_1V5 | = | 0 |
|  |  | ADC_VREF_2V5 | = | 1 |
|  |  | ADC_VREF_3V3 | = | 2 |
|  |  | ADC_VREF_External | = | 3 |

Return:

37

Returns zero on success. If an error occurs, a negative error code is returned.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.6.2 u2aADC_ConvertAndRead

int u2aADC_ConvertAndRead (U2A_HANDLE handle, Byte nBytes, Byte *Data) int CU2AClass::ADC_ConvertAndRead (Byte nBytes, Byte *Data)

This function triggers an ADC conversion and reads the results.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| nBytes | The size of the buffer for ADC result data – MUST be at least 12 bytes. |
| Data | Pointer to a buffer for the ADC result data. |

Notes:

The USB2ANY has four ADC channels. The inputs for channels zero (ADC0) and one (ADC1) have pre-amplifiers with a gain of two. So, the ADC result for signals on these pins will be a value that represents double the actual voltage of the signal on the pins. This makes these inputs compatible with smaller signals (maximum of VREF / 2).

The inputs for channels two (ADC2) and three (ADC3) do not have pre-amplifies, so the ADC result for signals on these pins represents the actual voltage of the signals.

The OneDemo does not have pre-amplifiers on any of the ADC inputs.

Return:

Returns a positive value on success. If an error occurs, a negative error code is returned. The results of the ADC conversion(s) are returned in the buffer pointed to by the Data parameter. The data in the buffer is formatted as follows:

38

| Byte | Value |
| --- | --- |
| 0 | Always equal to 26 |
| 1 | ADC0 result (High byte) |
| 2 | ADC0 result (Low byte) |
| 3 | ADC1 result (High byte) |
| 4 | ADC1 result (Low byte) |
| 5 | ADC2 result (High byte) |
| 6 | ADC2 result (Low byte) |
| 7 | ADC3 result (High byte) |
| 8 | ADC3 result (Low byte) |
| 9 | Reference voltage index (see VREF parameter of u2aADC_Control function) |

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.6.3 u2aADC_Enable

int u2aADC_Enable (U2A_HANDLE handle, int nChannel, int nMode)
int CU2AClass::ADC_Enable (int nChannel, int nMode)

This function enables or disables a specified ADC channel or range of channels. Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| nChannel | Specifies the ADC channel(s) to be enabled or disabled. When specifying a single channel, valid values are: 0 to 3 for USB2ANY, or 0 to 7 for OneDemo. |
| nMode | Defines whether the ADC channel is enabled or disabled: ADC_Disable = 0 Disables the specified single channel. ADC_Enable = 1 Enables the specified single channel. ADC_EnableMask = 2 Channels are enabled or disabled according to the bit mask specified by nChannel. For each channel, the ADC is enabled if the corresponding bit is set (1) or disabled if the bit is clear (0). |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.6.4 u2aADC_SetReference

int u2aADC_SetReference (U2A_HANDLE handle, ADC_VREF VREF)
int CU2AClass::ADC_SetReference (ADC_VREF VREF)

This function sets the voltage of the reference used for ADC conversion.
Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |  |
| --- | --- | --- |
| VREF | Sets the reference voltage for the ADC: |  |
|  | ADC_VREF_1V5 = | 0 |
|  | ADC_VREF_2V5 = | 1 |
|  | ADC_VREF_3V3 = | 2 |
|  | ADC_VREF_External = | 3 |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

39

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.6.5 u2aADC_Acquire

int u2aADC_Acquire (U2A_HANDLE handle, int nInterval, int nSamples) int CU2AClass::ADC_Acquire (int nInterval, int nSamples)

This function immediately triggers ADC conversions on the specified channel(s). A total of nSamples samples are acquired, with an interval of nInterval microseconds between samples.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| nInterval | The sampling interval, in microseconds. The interval must be in the range of 5 to 32767 microseconds. May not be less than 5 microseconds. |
| nSamples | Number of data points to acquire for each specified channel. If nSamples is set to one, the nInterval parameter is ignored. |

Notes:

Data is acquired for enabled channels (i.e., disabled channels are ignored) at each sample interval. See u2aADC_Enable for more about enabling/disabling channels.

There is a block of 4096 bytes of memory available to store the acquired data. Each data point requires two bytes of memory. The maximum number of samples per channel, per acquisition, is as follows:

| Channels enabled | Max. Samples per Channel |
| --- | --- |
| 1 | 2048 |
| 2 | 1024 |
| 3 | 682 |
| 4 | 512 |

| Channels enabled | Max. Samples per Channel |
| --- | --- |
| 5 | 409 |
| 6 | 341 |
| 7 | 292 |
| 8 | 256 |

Return:

On success, returns the number of data points (not samples) acquired. If an error occurs, a negative error code is returned. Use the u2aADC_GetData function to retrieve the actual data resulting from the conversion.

40

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.6.6 u2aADC_AcquireTriggered

int u2aADC_AcquireTriggered(U2A_HANDLE handle, int nInterval, int nSamples, int nTrigger)

int CU2AClass::ADC_AcquireTriggered(int nInterval, int nSamples, int nTrigger)

This function triggers ADC conversions on the specified channel(s) after the specified trigger event occurs. A total of nSamples samples are acquired, with an interval of nInterval microseconds between samples.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| nInterval | The sampling interval, in microseconds. May not be less than 5 microseconds. |
| nSamples | Number of data points to acquire for each specified channel. If nSamples is set to one, the nInterval parameter is ignored. |
| nTrigger | The event that will start the ADC conversion. Must be one of the following values: ADC_EVENT_I2C = 1 ADC starts after any I2C command is received, but before it is executed. |

Notes:

See u2aADC_Acquire Notes section for more information.

Return:

On success, returns the number of data points (not samples) acquired. If an error occurs, a

negative error code is returned.

5.6.7 u2aADC_GetData

int u2aADC_GetData (U2A_HANDLE handle, UInt16 nOffset, UInt16 nDataPoints, UInt16 *buffer)

41

int CU2AClass::ADC_GetData (UInt16 nOffset, UInt16 nDataPoints, UInt16 *buffer) This function retrieves the data acquired by the u2aADC_Acquire function.

Value 1 Value 2 Value 3 Value 4

...

Value nDataPoints

Channel 2 data

Channel 2 data

Channel 2 data

Channel 2 data

...

Channel 2 data

42

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| nOffset | Starting offset, in 16-bit words, of data to be transferred. |
| nDataPoints | Number of data points (16-bit words) to be transferred. If this number is larger than the available data, or is larger than the maximum allowable transfer size, the number of data points transferred may be less. In that case, the return value will reflect the actual number of data points that were transferred. |
| buffer | Address of a buffer to receive the transferred data. |

Notes:

The USB2ANY has four ADC channels. The inputs for channels zero (ADC0) and one (ADC1) have pre-amplifiers with a gain of two. So, the ADC result for signals on these pins will be a value that represents double the actual voltage of the signal on the pins. This makes these inputs compatible with smaller signals (maximum of VREF / 2).

The inputs for channels two (ADC2) and three (ADC3) do not have pre-amplifies, so the ADC result for signals on these pins represents the actual voltage of the signals.

The OneDemo does not have pre-amplifiers on any of the ADC inputs.

Comments:

The returned data is structured as nDataPoints records, with each record consisting of two bytes that represent a single 16-bit data point. The records are arranged as a repeating list of values for each enabled channel, in channel order. The u2aADC_Acquire function returns the number of converted data points, which can be used as the nDataPoints parameter to this function. The available number of data points can also be obtained by calling the u2aADC_GetStatus function.

For example, if only channel 2 is enabled, the returned data values would be in the following format:

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

If channels 1, 3, and 5 are enabled, the returned data would be in the following format:

Value 1 Value 2 Value 3 Value 4 Value 5 Value 6

...

Value nDataPoints

Channel 1 data

Channel 3 data

Channel 5 data

Channel 1 data

Channel 3 data

Channel 5 data

...

Channel 1 data

Channel 3 data

Channel 5 data

Return:

On success, returns the total number of data points copied to buffer. If an error occurs, a negative

error code is returned.

5.6.8 u2aADC_GetStatus

int using (U2A_HANDLE handle, BYTE *buffer)
int CU2AClass::ADC_GetStatus (BYTE *buffer)

This function retrieves the status of the ADC interface.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| buffer | Address of a buffer to receive the status information. The buffer must be large enough to hold at least 9 bytes of data. |

Return:

On success, returns the status of the ADC interface. If an error occurs, a negative error code is

returned.

43

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

The status is returned as follows:

44

| Byte | Meaning |  |  |  |
| --- | --- | --- | --- | --- |
| 0 |  | ADC channels enabled: |  |  |
|  |  | Bit | Meaning |  |
|  |  | 0 | ADC channel 0 is enabled |  |
|  |  | 1 | ADC channel 1 is enabled |  |
|  |  | 2 | ADC channel 2 is enabled |  |
|  |  | 3 | ADC channel 3 is enabled |  |
|  |  | 4 | ADC channel 4 is enabled |  |
|  |  | 5 | ADC channel 5 is enabled |  |
|  |  | 6 | ADC channel 6 is enabled |  |
|  |  | 7 | ADC channel 7 is enabled |  |
| 1 | Reference source index: ADC_VREF_1V5 = 0 ADC_VREF_2V5 = 1 ADC_VREF_3V3 = 2 ADC_VREF_External = 3 |  |  |  |
| 2 | Non-zero if capture is in progress |  |  |  |
| 3 | Non-zero if new data is available for transfer |  |  |  |
| 4 | Number of channels captured. |  |  |  |
| 5 | Low byte of data size (in 16-bit samples) |  |  |  |
| 6 | High byte of data size |  |  |  |
| 7 | Reserved (always zero) |  |  |  |
| 8 | Reserved (always zero) |  |  |  |

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.7	DAC Functions

5.7.1 u2aDACs_Write

int u2aDACs_Write (U2A_HANDLE handle, DACs_WhichDAC DACs_WhichDAC, DACs_OperatingMode DACs_OperatingMode, Byte Value)

int CU2AClass::DACs_Write (DACs_WhichDAC DACs_WhichDAC, DACs_OperatingMode DACs_OperatingMode, Byte Value)

This function writes data to the specified DAC.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| DACs_WhichDAC | Which DAC to write to: DAC0 = 0 DAC1 = 1 |
| DACs_OperatingMode | Sets the operating mode (ignored by OneDemo): DACs_Normal = 0 DACs_PWD_1k = 1 DACs_PWD_100k = 2 DACs_PWD_HiZ = 3 |
| Value | Value to write to the DAC. |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.7.2 u2aDAC_SetValue

int u2aDAC_SetValue (U2A_HANDLE handle, int WhichDAC, Byte Value) int CU2AClass::DAC_SetValue (int WhichDAC, Byte Value)

45

This function writes a value (to be converted to an output voltage) to the specified DAC.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| WhichDAC | Which DAC to write to: DAC0 = 0 DAC1 = 1 |
| Value | Value to write to the DAC. |

Comments:

The table below shows the correlation between Value and DAC output voltage. The voltages shown are approximate and depend on many factors (e.g., load current, 3.3V reference accuracy, etc.). The DAC outputs are intended to be used for very low current loads, such as voltage references. The outputs are capable of sourcing up to about 20ma, but deviation from the output voltage shown in the table may increase as the current load increases.

| Value | Volts | Value | Volts | Value | Volts | Value | Volts | Value | Volts | Value | Volts | Value | Volts | Value | Volts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00 | 0.000 | 20 | 0.414 | 40 | 0.828 | 60 | 1.242 | 80 | 1.656 | A0 | 2.071 | C0 | 2.485 | E0 | 2.899 |
| 01 | 0.013 | 21 | 0.427 | 41 | 0.841 | 61 | 1.255 | 81 | 1.669 | A1 | 2.084 | C1 | 2.498 | E1 | 2.912 |
| 02 | 0.026 | 22 | 0.440 | 42 | 0.854 | 62 | 1.268 | 82 | 1.682 | A2 | 2.096 | C2 | 2.511 | E2 | 2.925 |
| 03 | 0.039 | 23 | 0.453 | 43 | 0.867 | 63 | 1.281 | 83 | 1.695 | A3 | 2.109 | C3 | 2.524 | E3 | 2.938 |
| 04 | 0.052 | 24 | 0.466 | 44 | 0.880 | 64 | 1.294 | 84 | 1.708 | A4 | 2.122 | C4 | 2.536 | E4 | 2.951 |
| 05 | 0.065 | 25 | 0.479 | 45 | 0.893 | 65 | 1.307 | 85 | 1.721 | A5 | 2.135 | C5 | 2.549 | E5 | 2.964 |
| 06 | 0.078 | 26 | 0.492 | 46 | 0.906 | 66 | 1.320 | 86 | 1.734 | A6 | 2.148 | C6 | 2.562 | E6 | 2.976 |
| 07 | 0.091 | 27 | 0.505 | 47 | 0.919 | 67 | 1.333 | 87 | 1.747 | A7 | 2.161 | C7 | 2.575 | E7 | 2.989 |
| 08 | 0.104 | 28 | 0.518 | 48 | 0.932 | 68 | 1.346 | 88 | 1.760 | A8 | 2.174 | C8 | 2.588 | E8 | 3.002 |
| 09 | 0.116 | 29 | 0.531 | 49 | 0.945 | 69 | 1.359 | 89 | 1.773 | A9 | 2.187 | C9 | 2.601 | E9 | 3.015 |
| 0A | 0.129 | 2A | 0.544 | 4A | 0.958 | 6A | 1.372 | 8A | 1.786 | AA | 2.200 | CA | 2.614 | EA | 3.028 |
| 0B | 0.142 | 2B | 0.556 | 4B | 0.971 | 6B | 1.385 | 8B | 1.799 | AB | 2.213 | CB | 2.627 | EB | 3.041 |
| 0C | 0.155 | 2C | 0.569 | 4C | 0.984 | 6C | 1.398 | 8C | 1.812 | AC | 2.226 | CC | 2.640 | EC | 3.054 |
| 0D | 0.168 | 2D | 0.582 | 4D | 0.996 | 6D | 1.411 | 8D | 1.825 | AD | 2.239 | CD | 2.653 | ED | 3.067 |
| 0E | 0.181 | 2E | 0.595 | 4E | 1.009 | 6E | 1.424 | 8E | 1.838 | AE | 2.252 | CE | 2.666 | EE | 3.080 |
| 0F | 0.194 | 2F | 0.608 | 4F | 1.022 | 6F | 1.436 | 8F | 1.851 | AF | 2.265 | CF | 2.679 | EF | 3.093 |
| 10 | 0.207 | 30 | 0.621 | 50 | 1.035 | 70 | 1.449 | 90 | 1.864 | B0 | 2.278 | D0 | 2.692 | F0 | 3.106 |
| 11 | 0.220 | 31 | 0.634 | 51 | 1.048 | 71 | 1.462 | 91 | 1.876 | B1 | 2.291 | D1 | 2.705 | F1 | 3.119 |
| 12 | 0.233 | 32 | 0.647 | 52 | 1.061 | 72 | 1.475 | 92 | 1.889 | B2 | 2.304 | D2 | 2.718 | F2 | 3.132 |
| 13 | 0.246 | 33 | 0.660 | 53 | 1.074 | 73 | 1.488 | 93 | 1.902 | B3 | 2.316 | D3 | 2.731 | F3 | 3.145 |
| 14 | 0.259 | 34 | 0.673 | 54 | 1.087 | 74 | 1.501 | 94 | 1.915 | B4 | 2.329 | D4 | 2.744 | F4 | 3.158 |
| 15 | 0.272 | 35 | 0.686 | 55 | 1.100 | 75 | 1.514 | 95 | 1.928 | B5 | 2.342 | D5 | 2.756 | F5 | 3.171 |
| 16 | 0.285 | 36 | 0.699 | 56 | 1.113 | 76 | 1.527 | 96 | 1.941 | B6 | 2.355 | D6 | 2.769 | F6 | 3.184 |
| 17 | 0.298 | 37 | 0.712 | 57 | 1.126 | 77 | 1.540 | 97 | 1.954 | B7 | 2.368 | D7 | 2.782 | F7 | 3.196 |
| 18 | 0.311 | 38 | 0.725 | 58 | 1.139 | 78 | 1.553 | 98 | 1.967 | B8 | 2.381 | D8 | 2.795 | F8 | 3.209 |
| 19 | 0.324 | 39 | 0.738 | 59 | 1.152 | 79 | 1.566 | 99 | 1.980 | B9 | 2.394 | D9 | 2.808 | F9 | 3.222 |
| 1A | 0.336 | 3A | 0.751 | 5A | 1.165 | 7A | 1.579 | 9A | 1.993 | BA | 2.407 | DA | 2.821 | FA | 3.235 |
| 1B | 0.349 | 3B | 0.764 | 5B | 1.178 | 7B | 1.592 | 9B | 2.006 | BB | 2.420 | DB | 2.834 | FB | 3.248 |
| 1C | 0.362 | 3C | 0.776 | 5C | 1.191 | 7C | 1.605 | 9C | 2.019 | BC | 2.433 | DC | 2.847 | FC | 3.261 |
| 1D | 0.375 | 3D | 0.789 | 5D | 1.204 | 7D | 1.618 | 9D | 2.032 | BD | 2.446 | DD | 2.860 | FD | 3.274 |
| 1E | 0.388 | 3E | 0.802 | 5E | 1.216 | 7E | 1.631 | 9E | 2.045 | BE | 2.459 | DE | 2.873 | FE | 3.287 |
| 1F | 0.401 | 3F | 0.815 | 5F | 1.229 | 7F | 1.644 | 9F | 2.058 | BF | 2.472 | DF | 2.886 | FF | 3.300 |

Value vs. DAC output voltage

Return:

46

Returns zero on success. If an error occurs, a negative error code is returned

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.8	PWM Functions

5.8.1 u2aPWM_Control

int u2aPWM_Control (U2A_HANDLE handle, PWM_ModeControl ModeControl, PWM_WhichPWM PWM_WhichPWM,

PWM_InputDivider InputDivider, UInt16 CompareRegister0, PWM_OutputMode OutputMode1, UInt16 CompareRegister1, PWM_InputDividerEX InputDividerEX)

int CU2AClass::PWM_Control (PWM_ModeControl ModeControl,
PWM_WhichPWM PWM_WhichPWM,

PWM_InputDivider InputDivider, UInt16 CompareRegister0, PWM_OutputMode OutputMode1, UInt16 CompareRegister1, PWM_InputDividerEX InputDividerEX)

This function sets the parameters for the PWM pins.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |  |
| --- | --- | --- |
| ModeControl | Sets the mode of the selected PWM channel: |  |
|  | PWM_Stop | = 0 |
|  | PWM_Up | = 1 |
|  | PWM_Continuous | = 2 |
|  | PWM_UP_Down | = 3 |
| PWM_WhichPWM | Selects the PWM channel: |  |
|  | PWM0 | = 0 |
|  | PWM1 | = 1 |
|  | PWM2 | = 2 |
|  | PWM3 | = 3 |
| InputDivider | Sets the divider for the selected PWM channel: |  |
|  | PWM_Div_1 | = 0 (24 MHz) |
|  | PWM_Div_2 | = 1 (12 MHz) |
|  | PWM_Div_4 | = 2 (6 MHz) |
|  | PWM_Div_8 | = 3 (3 MHz) |
|  | This determines the frequency at which the timer counts. |  |
| CompareRegister0 | Frequency control (sets frequency). Timer counts up to this value, then counts down to 0. |  |

47

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

| OutputMode1 | Sets the output mode for the selected PWM channel: PWM_Out_bit_value = 0 PWM_Out_Set = 1 PWM_Out_Toggle_Reset = 2 PWM_Out_Set_Reset = 3 PWM_Out_Toggle = 4 PWM_Out_Reset = 5 PWM_Out_Toggle_Set = 6 PWM_Out_Reset_Set = 7 |
| --- | --- |
| CompareRegister1 | PWM control (sets duty cycle). This is the value of the timer count at which the output toggles. |
| InputDividerEX | Further divides the input clock. This must be one of the following values: |
|  | 0 = /1 |
|  | 1 = /2 |
|  | 2 = /3 |
|  | 3 = /4 |
|  | 4 = /5 |
|  | 5 = /6 |
|  | 6 = /7 |
|  | 7 = /8 |
|  | If set to a non-zero value, the actual input divider is the InputDivider divider times the InputDividerEX divider. For example, if you set |
|  | InputDivider to 2 and InputDividerEX to 5, the input frequency would be calculated as follows: |
|  | FREQIN = 24 MHz / (4 * 6) = 24 MHz / 24 = 1MHz |

48

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Return:

49

Returns zero on success. If an error occurs, a negative error code is returned.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.9	UART Functions

5.9.1 u2aUART_Control

int u2aUART_Control (U2A_HANDLE handle, UInt16 BaudRate, UInt16 Parity,

UInt16 BitDirection, UInt16 CharacterLength, UInt16 StopBits)

int CU2AClass::UART_Control (UInt16 BaudRate, UInt16 Parity, UInt16 BitDirection, UInt16 CharacterLength, UInt16 StopBits)

This function sets the USART parameters for serial communication.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |  |
| --- | --- | --- |
| BaudRate | Sets the baud rate for the USART transmitter and receiver: |  |
|  | UART_9600_bps = | 0 |
|  | UART_19200_bps = | 1 |
|  | UART_38400_bps = | 2 |
|  | UART_57600_bps = | 3 |
|  | UART_115200_bps = | 4 |
|  | UART_230400_bps = | 5 |
|  | UART_300_bps = | 6 |
|  | UART_320_bps = | 7 |
|  | UART_600_bps = | 8 |
|  | UART_1200_bps = | 9 |
|  | UART_2400_bps = | 10 |
|  | UART_4800_bps = | 11 |
| Parity | Sets the parity encoding/decoding: |  |
|  | UART_None = | 0 |
|  | UART_Even = | 1 |
|  | UART_Odd = | 2 |
| BitDirection | Defines which bit of each byte is sent first: |  |
|  | UART_LSB_First = 0 (recommended) |  |
|  | UART_MSB_First = 1 (non-standard) |  |
| CharacterLength | Sets the number of data bits in each character: |  |
|  | UART_8_Bit = | 0 |
|  | UART_7_Bit = | 1 |
| StopBits | Sets the number of stop bits: |  |
|  | UART_One_Stop = | 0 |
|  | UART_Two_Stop = | 1 |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

50

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.9.2 u2aUART_Write

int u2aUART_Write (U2A_HANDLE handle, Byte nBytes, Byte *Data)
int CU2AClass::UART_Write (Byte nBytes, Byte *Data)

This function writes serial data via the USART.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| nBytes | The number of bytes to be written. |
| Data | Pointer to array of bytes to be written. |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.9.3 u2aUART_Read

int u2aUART_Read (U2A_HANDLE handle, Byte nBytes, Byte *Data)
int CU2AClass::UART_Read (Byte nBytes, Byte *Data)

This function reads serial data via the USART.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| nBytes | The number of bytes to be read. |
| Data | Pointer to array of bytes to receive the data read. |

Return:

On success, returns the number of bytes read. If an error occurs, a negative error code is returned.

5.9.4 u2aUART_DisableReceiver

int u2aUART_DisableReceiver (U2A_HANDLE handle)
int CU2AClass::UART_DisableReceiver ()

This function disables the reception of serial data via the USART. Once this function is executed, the serial data receiver will remain disabled until the u2aUART_Control function is called.

51

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.9.5 u2aUART_GetRxCount

int u2aUART_GetRxCount (U2A_HANDLE handle)
int CU2AClass::UART_GetRxCount ()

This function checks the receive queue for data received by the UART and returns the number of data bytes that are currently available to be read.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |

Return:

On success, returns the number of bytes available to be read (possibly zero). If an error occurs, a negative error code is returned.

5.9.6 u2aUART_SetMode

int u2aUART_SetMode (U2A_HANDLE handle, UInt16 mode)
int CU2AClass::UART_SetMode (UInt16 mode)

This is used to allow the UART functions to operate in special modes.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |  |
| --- | --- | --- |
| mode | One of the follow mode constants: |  |
|  | UART_Normal = | 0 |
|  | UART_ReceiverOff = | 1 |
|  | UART_RecvAfterXmit = | 2 |

Return:

52

Returns zero on success. If an error occurs, a negative error code is returned.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.10	GPIO Functions

5.10.1 u2aGPIO_WriteControl

int u2aGPIO_WriteControl (U2A_HANDLE handle, GPIO_PinFunction GPIO0, GPIO_PinFunction GPIO1, GPIO_PinFunction GPIO2, GPIO_PinFunction GPIO3, GPIO_PinFunction GPIO4, GPIO_PinFunction GPIO5, GPIO_PinFunction GPIO6, GPIO_PinFunction GPIO7, GPIO_PinFunction GPIO8, GPIO_PinFunction GPIO9, GPIO_PinFunction GPIO10, GPIO_PinFunction GPIO11, GPIO_PinFunction GPIO12)

int CU2AClass::GPIO_WriteControl (GPIO_PinFunction GPIO0, GPIO_PinFunction GPIO1, GPIO_PinFunction GPIO2, GPIO_PinFunction GPIO3, GPIO_PinFunction GPIO4, GPIO_PinFunction GPIO5, GPIO_PinFunction GPIO6, GPIO_PinFunction GPIO7, GPIO_PinFunction GPIO8, GPIO_PinFunction GPIO9, GPIO_PinFunction GPIO10, GPIO_PinFunction GPIO11, GPIO_PinFunction GPIO12)

This function writes control data to all of the GPIO pins, simultaneously.
Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| GPIO0 | Function of GPIO pin 0 |
| GPIO1 | Function of GPIO pin 1 |
| GPIO2 | Function of GPIO pin 2 |
| GPIO3 | Function of GPIO pin 3 |
| GPIO4 | Function of GPIO pin 4 |
| GPIO5 | Function of GPIO pin 5 |
| GPIO6 | Function of GPIO pin 6 |
| GPIO7 | Function of GPIO pin 7 |
| GPIO8 | Function of GPIO pin 8 |
| GPIO9 | Function of GPIO pin 9 |
| GPIO10 | Function of GPIO pin 10 |
| GPIO11 | Function of GPIO pin 11 |
| GPIO12 | Function of GPIO pin 12 |

Comments:

The following constant values are used to define the function of each GPIO pin:

| Constant | Value | GPIO Pin Function |
| --- | --- | --- |
| GPIO_No_Change | 0 | The pin’s function is not changed. |
| GPIO_Output | 1 | Sets pin as an output. |
| GPIO_Input_No_Resistor | 2 | Sets pin as a floating input with no resistor. |
| GPIO_Input_Pull_Up | 3 | Sets pin as an input with a pull-up resistor. |
| GPIO_Input_Pull_Down | 4 | Sets pin as an input with a pull-down resistor. |

53

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.10.2 u2aGPIO_WriteState

int u2aGPIO_WriteState (U2A_HANDLE handle, GPIO_OutPinState GPIO0,

GPIO_OutPinState GPIO1, GPIO_OutPinState GPIO2, GPIO_OutPinState GPIO3, GPIO_OutPinState GPIO4, GPIO_OutPinState GPIO5, GPIO_OutPinState GPIO6, GPIO_OutPinState GPIO7, GPIO_OutPinState GPIO8, GPIO_OutPinState GPIO9, GPIO_OutPinState GPIO10, GPIO_OutPinState GPIO11, GPIO_OutPinState GPIO12)

int CU2AClass::GPIO_WriteState (GPIO_OutPinState GPIO0, GPIO_OutPinState GPIO1, GPIO_OutPinState GPIO2, GPIO_OutPinState GPIO3, GPIO_OutPinState GPIO4, GPIO_OutPinState GPIO5, GPIO_OutPinState GPIO6, GPIO_OutPinState GPIO7, GPIO_OutPinState GPIO8, GPIO_OutPinState GPIO9, GPIO_OutPinState GPIO10, GPIO_OutPinState GPIO11, GPIO_OutPinState GPIO12)

This function sets the output state of all GPIO pins, simultaneously.
Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| GPIO0 | Output state of GPIO pin 0 |
| GPIO1 | Output state of GPIO pin 1 |
| GPIO2 | Output state of GPIO pin 2 |
| GPIO3 | Output state of GPIO pin 3 |
| GPIO4 | Output state of GPIO pin 4 |
| GPIO5 | Output state of GPIO pin 5 |
| GPIO6 | Output state of GPIO pin 6 |
| GPIO7 | Output state of GPIO pin 7 |
| GPIO8 | Output state of GPIO pin 8 |
| GPIO9 | Output state of GPIO pin 9 |
| GPIO10 | Output state of GPIO pin 10 |
| GPIO11 | Output state of GPIO pin 11 |
| GPIO12 | Output state of GPIO pin 12 |

Comments:

The following constant values are used to define the output state of each GPIO pin:

| Constant | Value | GPIO Pin Function |
| --- | --- | --- |
| GPIO_Out_No_Change | 0 | The pin’s output is not changed. |
| GPIO_Out_Low | 1 | Sets pin’s output to a low state. |
| GPIO_Out_High | 2 | Sets pin’s output to a high state. |

54

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Important Note: This function affects only those pins that are configured as outputs.

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.10.3 u2aGPIO_ReadState

int u2aGPIO_ReadState (U2A_HANDLE handle, Byte nBytes, Byte *Data) int CU2AClass::GPIO_ReadState (Byte nBytes, Byte *Data)

This function reads the input state of all of the GPIO pins, simultaneously.
Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| nBytes | The size of the buffer to receive the data. Size must be at least 13 bytes to receive data for all GPIO pins. |
| Data | The address of a buffer to receive the data. |

Important Note: This function returns valid values only for those pins that are configured as inputs. Data is received for pins set as outputs, but it is meaningless.

Return:

Returns number of bytes copied to the Data buffer on success. If an error occurs, a negative error

code is returned.

Data is returned as an array of bytes. The first byte is the value read from GPIO0, the second byte is read from GPIO1, and so on. Data bytes are returned for all GPIO pins, regardless of whether they are configured as inputs. See the Important Note above.

5.10.4 u2aGPIO_SetPort

int u2aGPIO_SetPort (U2A_HANDLE handle, Byte GPIO_Port, Byte function) int CU2AClass::GPIO_SetPort (Byte GPIO_Port, Byte function)

This function configures a single GPIO pin as an output or input (with resistor options). Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| GPIO_Port | The GPIO port to be configured. Must be in the range 0 – 12. |
| function | A function code from the table below. |

55

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

The function parameter must be set to one of the following constant values:

| Constant | Value | GPIO Pin Function |
| --- | --- | --- |
| GPIO_No_Change | 0 | The pin’s function is not changed. |
| GPIO_Output | 1 | Sets pin as an output. |
| GPIO_Input_No_Resistor | 2 | Sets pin as a floating input with no resistor. |
| GPIO_Input_Pull_Up | 3 | Sets pin as an input with a pull-up resistor. |
| GPIO_Input_Pull_Down | 4 | Sets pin as an input with a pull-down resistor. |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.10.5 u2aGPIO_WritePort

int u2aGPIO_WritePort (U2A_HANDLE handle, Byte GPIO_Port, Byte state) int CU2AClass::GPIO_WritePort (Byte GPIO_Port, Byte state)

This function sets the state of a single GPIO output pin.
Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| GPIO_Port | The GPIO port to be configured. Must be in the range 0 – 12. |
| state | A state constant from the table below. |

The state parameter must be set to one of the following constant values:

| Constant | Value | GPIO Pin Function |
| --- | --- | --- |
| No_Change | 0 | The pin’s output is not changed. |
| Low | 1 | Sets pin’s output to a low state. |
| High | 2 | Sets pin’s output to a high state. |

Important Note: This function has no effect unless the specified pin is configured as an output.

Return:

Returns zero on success, or a negative error code on failure. Returns

56

ERR_INVALID_CONFIGURATION if the specified pin is not configured as an output.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.10.6 u2aGPIO_ReadPort

int u2aGPIO_ReadPort (U2A_HANDLE handle, Byte GPIO_Port)
int CU2AClass::GPIO_ReadPort (Byte GPIO_Port)

This function reads the state of a single GPIO input pin.
Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| GPIO_Port | The GPIO port to be configured. Must be in the range 0 – 12. |

Important Note: This function has no effect unless the specified pin is configured as an output.

Return:

On success, returns zero if the input pin is low, one if it is high. On error, a negative error code is

returned. Returns ERR_INVALID_CONFIGURATION if the specified pin is not configured as an input.

5.10.7 u2aGPIO_WritePulse

int u2aGPIO_WritePulse (U2A_HANDLE handle, Byte GPIO_Port, Byte polarity, UInt16 duration)

int CU2AClass::GPIO_WritePulse (Byte GPIO_Port, Byte polarity, UInt16 duration)

This function outputs a single pulse on a specified GPIO output pin. The pulse can be either “high” or “low” with a width of 5 to 65535 microseconds

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| GPIO_Port | The GPIO port to be used for output. Must be in the range 0 – 12. |
| polarity | The polarity of the pulse: 0 = low pulse, 1 = high pulse. |
| duration | The desired pulse width, in microseconds. Must be at least 5 microseconds. May be set to zero to initialize the output state, without producing a pulse. |

Comments:

This function will automatically configure the specified port as an output and set the initial state to the opposite of the polarity parameter.

For best results, the pin specified by the GPIO_Port parameter should be preset to the correct initial state by calling this function with the polarity parameter set to the desired value and the duration parameter set to zero. Otherwise, if the initial state is incorrect, an unexpected extra edge may be produced.

The timing of the pulse is uncalibrated, but is typically accurate to within 0.5%.

57

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Example A: polarity = 1, duration = 14

Example B: polarity = 0, duration = 2200

Return:

58

Returns zero on success, or a negative error code on failure. Returns the error code ERR_INVALID_CONFIGURATION if the specified pin is not valid, or the specified pulse width is less than 5 microseconds.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.11	Memory Functions

5.11.1 u2aMSP430_ByteRead

int u2aMSP430_ByteRead (U2A_HANDLE handle, UInt16 Address)
int CU2AClass::MSP430_ByteRead (UInt16 Address)

This function reads a single byte from the MSP430 memory.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| Address | The 16-bit address of the byte to read. |

Return:

On success, returns the unsigned value of the byte read. If an error occurs, a negative error code is

returned.

5.11.2 u2aMSP430_ByteWrite

int u2aMSP430_ByteWrite (U2A_HANDLE handle, UInt16 Address, Byte Value) int CU2AClass::MSP430_ByteWrite (UInt16 Address, Byte Value)

This function writes a single byte to the MSP430 memory.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| Address | The 16-bit address of the byte to write. |
| Value | The 8-bit value to write. |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.11.3 u2aMSP430_WordRead

int u2aMSP430_WordRead (U2A_HANDLE handle, UInt16 Address)
int CU2AClass::MSP430_WordRead (UInt16 Address)

This function reads a single word from the MSP430 memory.

59

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| Address | The 16-bit address of the word to read. |

Return:

On success, returns the unsigned value of the word read. If an error occurs, a negative error code

is returned.

5.11.4 u2aMSP430_WordWrite

int u2aMSP430_WordWrite (U2A_HANDLE handle, UInt16 Address, UInt16 Value) int CU2AClass::MSP430_WordWrite (UInt16 Address, UInt16 Value)

This function writes a single word to the MSP430 memory.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| Address | The 16-bit address of the word to write. |
| Value | The 16-bit value to write. |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.11.5 u2aMSP430_MemoryRead

int u2aMSP430_MemoryRead (U2A_HANDLE handle, UInt16 Address, UInt16 nBytes, Byte *Data)

int CU2AClass::MSP430_MemoryRead (UInt16 Address, UInt16 nBytes, Byte *Data) This function reads a user-defined number of bytes from the MSP430 memory.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| Address | The 16-bit address of the first byte of MSP430 memory to read. |
| nBytes | The number of bytes to be read. Must be in the range of 1-54 bytes. |
| Data | Pointer to a buffer to receive the bytes read. |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

60

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.11.6 u2aMSP430_MemoryWrite

int u2aMSP430_MemoryWrite (U2A_HANDLE handle, UInt16 Address, UInt16 nBytes, Byte *Data)

int CU2AClass::MSP430_MemoryWrite (UInt16 Address, UInt16 nBytes, Byte *Data)

This function writes a user-defined number of bytes to the MSP430 memory.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| Address | The 16-bit address of the first byte of MSP430 memory to write. |
| nBytes | The number of bytes to be written. Must be in the range of 1-54 bytes. |
| Data | Pointer to a buffer of bytes to be written. |

Return:

61

Returns zero on success. If an error occurs, a negative error code is returned.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.12	FEC Functions

5.12.1 u2aFEC_Control (Deprecated)

int u2aFEC_Control (U2A_HANDLE handle, UInt16 Interval, UInt16 Enable, UInt16 Divider0, UInt16 Divider1)

Warning: This function is deprecated. Use u2aFEC_Configure for all new development.

This function configures the Frequency and Event Counter (FEC).

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| Interval | A number representing the desired sample interval. See the table below. |
| Enable | Set to one (1) to enable FEC, zero (0) to disable it. |
| Divider0 | Value for ID (input divider) register. See MSP430F5529 User’s Guide. |
| Divider1 | Value for TAIDEX register. See MSP430F5529 User’s Guide. |

Comments:

The Interval parameter must be a value from the following table:

| Interval | Sample period |
| --- | --- |
| 0 | 1 millisecond |
| 1 | 2 milliseconds |
| 2 | 5 milliseconds |
| 3 | 10 milliseconds |
| 4 | 20 milliseconds |
| 5 | 50 milliseconds |
| 6 | 100 milliseconds |
| 7 | 200 milliseconds |
| 8 | 500 milliseconds |
| 9 | 1 second |
| 10 | 2 seconds |
| 11 | 5 seconds |
| 12 | 10 seconds |

Return:

62

Returns zero on success. If an error occurs, a negative error code is returned.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.12.2 u2aFEC_Configure

int u2aFEC_Configure (U2A_HANDLE handle, UInt16 Enable, UInt16 Interval, UInt16 Divider)

int CU2AClass::FEC_Configure (UInt16 Enable, UInt16 Interval, UInt16 Divider) This function configures the Frequency and Event Counter (FEC).

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| Enable | Set to one (1) to enable FEC, zero (0) to disable it. |
| Interval | A number representing the desired sample interval. See the table below. |
| Divider | The desired input frequency divider. See the table below. |

Comments:

The Interval parameter must be a value from the following table:

| Interval | Sample period | Interval | Sample period |
| --- | --- | --- | --- |
| 0 | 1 millisecond | 7 | 200 milliseconds |
| 1 | 2 milliseconds | 8 | 500 milliseconds |
| 2 | 5 milliseconds | 9 | 1 second |
| 3 | 10 milliseconds | 10 | 2 seconds |
| 4 | 20 milliseconds | 11 | 5 seconds |
| 5 | 50 milliseconds | 12 | 10 seconds |
| 6 | 100 milliseconds |  |  |

The Divider parameter must be one of the values from the following table:

| 1 | 6 | 14 | 32 |
| --- | --- | --- | --- |
| 2 | 7 | 16 | 40 |
| 3 | 8 | 20 | 48 |
| 4 | 10 | 24 | 56 |
| 5 | 12 | 28 | 64 |

Return:

63

Returns zero on success. If an error occurs, a negative error code is returned.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.12.3 u2aFEC_CountAndRead

int u2aFEC_CountAndRead (U2A_HANDLE handle)

int CU2AClass::FEC_CountAndRead ()

This function starts the Frequency and Event Counter (FEC) and then attempts to retrieve the result.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |

Comments:

About 20 milliseconds after starting the FEC, an attempt is made to retrieve the result. If the

attempt is unsuccessful, a second attempt is made after a delay of 25 milliseconds. If both attempts fail, an error code of ERR_NO_DATA is returned.

The u2aFEC_CountAndRead function will generally return the ERR_NO_DATA code when the Interval parameter passed to the u2aFEC_Configure function specifies an interval greater than 20 milliseconds. In that case, you can retrieve the result by calling the u2aFEC_GetResult function after the time period specified by the Interval parameter has passed. The u2aFEC_CountAndRead function was designed in this way to allow longer intervals to be handled asynchronously.

Return:

On success, returns the number of events (may be zero) counted during the specified time interval. If an error occurs, a negative error code is returned. See Comments section for an explanation of the ERR_NO_DATA error code.

If the input to the FEC is a periodic waveform, you can calculate the frequency (in Hz), using the following formula:

freqHz = (1.0 / Intervalsecs) * ReturnValue

5.12.4 u2aFEC_PurgeBuffer

int u2aFEC_PurgeBuffer (U2A_HANDLE handle)

int CU2AClass::FEC_PurgeBuffer ()

64

This function purges any un-retrieved results from the FEC results buffer.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.12.5 u2aFEC_GetResult

int u2aFEC_GetResult (U2A_HANDLE handle)

int CU2AClass::FEC_GetResult ()

This function retrieves the result a call to the u2aFEC_CountAndRead function.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |

Return:

On success, returns the result (event count) of the u2aFEC_CountAndRead function call. If an

error occurs, a negative error code is returned.

Comments:

This function should be called only if a call to the u2aFEC_CountAndRead function returned an error code of ERR_NO_DATA. If the u2aFEC_CountAndRead function returns a valid event count, this function will always return the ERR_NO_DATA code.

65

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.13	Interrupt Functions

5.13.1 u2aInterrupt_Control

int u2aInterrupt_Control (U2A_HANDLE handle, UInt16 Interrupt_PinFunction0,

UInt16 Interrupt_PinFunction1,

UInt16 Interrupt_PinFunction2,

UInt16 Interrupt_PinFunction3)

int CU2AClass::Interrupt_Control (UInt16 Interrupt_PinFunction0,

UInt16 Interrupt_PinFunction1,

UInt16 Interrupt_PinFunction2,

UInt16 Interrupt_PinFunction3)

This function is used to enable, disable, and set the parameters of the four interrupt pins on the USB2ANY.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| Interrupt_PinFunction0 | Sets the state of interrupt pin 0 (INT0). Not available on OneDemo. |
| Interrupt_PinFunction1 | Sets the state of interrupt pin 1 (INT1). |
| Interrupt_PinFunction2 | Sets the state of interrupt pin 2 (INT2). |
| Interrupt_PinFunction3 | Sets the state of interrupt pin 3 (INT3). |

Comments:

For each interrupt pin, the state is set according to the following table:

| Parameter | Value | Description |
| --- | --- | --- |
| Intr_No_Change | 0 | No change |
| Intr_Falling_Edge | 1 | Interrupt enabled on falling edge |
| Intr_Rising_Edge | 2 | Interrupt enabled on rising edge |
| Intr_Disabled | 3 | Interrupt disabled |

Note: INT0 is not available on OneDemo because the pin is used for EVM Detect.

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.13.2 u2aInterrupt_CheckReceived

int u2aInterrupt_CheckReceived (U2A_HANDLE handle, int Channel, BOOL Reset) int CU2AClass::Interrupt_CheckReceived (int Channel, BOOL Reset)

66

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

This function is used to check whether an interrupt has occurred on the specified channel.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| Channel | The interrupt channel to check (0=INT0, 1 = INT1, 2=INT2, 3=INT3) |
| Reset | If set TRUE, resets the count for the specified channel to zero, after the current count is read. If set FALSE, the count will continue to increment for each interrupt received. |

Return:

Returns the number of interrupts received for the specified channel. If an error occurs, a negative

error code is returned.

5.13.3 u2aInterrupt_SetHandler

int u2aInterrupt_SetHandler (U2A_HANDLE handle, int Channel, void *Callback, HWND Hwnd, int MsgNum)

int CU2AClass::Interrupt_SetHandler (int Channel, void *Callback, HWND Hwnd, int MsgNum)

This function is used to configure the notification method used by the interrupt handler. Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| Channel | The interrupt channel to configure (0=INT0, 1 = INT1, 2=INT2, 3=INT3) |
| Callback | The address of the callback function to be called when the specified interrupt occurs. If set to NULL, no function is called. See Comments section for more information. |
| Hwnd | The handle of the window to receive a notification message when the specified interrupt occurs. If set to NULL, no message is sent. See Comments section for more information. |
| MsgNum | The message number to be sent to the window specified by Hwnd. |

Comments:

When Callback is set to a non-Null value, a function at the specified address will be called when

an interrupt occurs on the specified Channel. That function must have the following prototype:

void _stdcall CallBack(DWORD channel, DWORD count)

There will be two parameters passed to the function. The first parameter, channel, will be set to the channel number of the interrupt (i.e., 0 – 3). This allows multiple interrupts to share the same callback function.

67

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

The second parameter, count, is set to the number of interrupts that occurred since the last call to the callback function. Normally, this will be set to one. However, if additional interrupts occur before the callback function returns, they will be accumulated and reported by the next call to the callback function. To avoid accumulation of interrupts, the callback function should be written such that it processes each call in the minimum amount of time, with minimum overhead.

When Hwnd is set to a non-Null value, a message will be posted to the specified window’s queue. The message will be in the standard Windows message format, with two parameters: wParam and lParam. The wParam parameter will be set to the interrupt channel number and the lParam parameter will be set to the interrupt count, as with the callback function.

If both Callback and Hwnd are set to NULL, the u2aInterrupt_CheckReceived function may be used to determine whether interrupts have occurred, using a polling method. Note that, if desired, both Callback and Hwnd methods can be used simultaneously.

To turn off either (or both) of the Callback and Hwnd methods, call this function with the appropriate parameter(s) set to NULL, always making sure that both parameters are set to the desired values.

Return:

68

Returns zero on success. If an error occurs, a negative error code is returned.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.14	Digital Capture Functions

5.14.1 u2aDigital_Capture

int u2aDigital_Capture (U2A_HANDLE handle, UInt32 frequency,
UInt16 samples, UInt16 timeframe)

int CU2AClass::Digital_Capture (UInt32 frequency, UInt16 samples,

UInt16 timeframe)

This function is used to initiate capture of digital data via the GPIO7 input pin.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |  |  |  |
| --- | --- | --- | --- | --- |
| frequency | The sampling frequency in Hertz. Must be in the range of 16 Hz to |  |  |  |
|  | 262143 Hz. |  |  |  |
| timeframe | A number representing the length of the time window, during which the |  |  |  |
|  | digital signal is sampled. | Must be one of the following |  | values: |
|  |  | timeframe | Capture time |  |
|  |  |  | (milliseconds) |  |
|  |  | 0 | 1 |  |
|  |  | 1 | 2 |  |
|  |  | 2 | 5 |  |
|  |  | 3 | 10 |  |
|  |  | 4 | 20 |  |
|  |  | 5 | 50 |  |
|  |  | 6 | 100 |  |
|  |  | 7 | 200 |  |
|  |  | 8 | 500 |  |
|  |  | 9 | 1000 |  |
|  |  | 10 | 2000 |  |
|  |  | 11 | 5000 |  |
|  |  | 12 | 10000 |  |

Comments:

The actual sampling frequency is derived by dividing the USB2ANY’s master clock by an integer value. This limits the number of actual frequencies available. The actual frequency used for the sampling interval is calculated by an algorithm that chooses a frequency at least as high as the requested frequency (possibly much higher). The difference between the requested and actual frequencies is much more obvious at higher frequencies, where the clock divider is a small number. For example, requesting any frequency over 250,000 Hz will result in the maximum sampling frequency of 333,333 Hz.

69

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Return:

On success, returns the actual sampling frequency (see Comments section for more information).

If an error occurs, a negative error code is returned.

70

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.15	EasyScaleTM Functions

5.15.1 u2aEasyScale_Control

int u2aEasyScale_Control (U2A_HANDLE handle, UInt16 UpperThreshold, UInt16 LowerThreshold)

int CU2AClass::EasyScale_Control (UInt16 UpperThreshold, UInt16 LowerThreshold)

This function is used to set the voltage thresholds for EasyScaleTM communications. Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| UpperThreshold | The upper voltage threshold used for determining data states. |
| LowerThreshold | The lower voltage threshold used for determining data states. |

Comments:

The table below shows the values available for the UpperThreshold and LowerThreshold parameters and the approximate voltage they represent:

| Value | Volts | Value | Volts | Value | Volts | Value | Volts | Value | Volts | Value | Volts | Value | Volts | Value | Volts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00 | 0.000 | 20 | 0.414 | 40 | 0.828 | 60 | 1.242 | 80 | 1.656 | A0 | 2.071 | C0 | 2.485 | E0 | 2.899 |
| 01 | 0.013 | 21 | 0.427 | 41 | 0.841 | 61 | 1.255 | 81 | 1.669 | A1 | 2.084 | C1 | 2.498 | E1 | 2.912 |
| 02 | 0.026 | 22 | 0.440 | 42 | 0.854 | 62 | 1.268 | 82 | 1.682 | A2 | 2.096 | C2 | 2.511 | E2 | 2.925 |
| 03 | 0.039 | 23 | 0.453 | 43 | 0.867 | 63 | 1.281 | 83 | 1.695 | A3 | 2.109 | C3 | 2.524 | E3 | 2.938 |
| 04 | 0.052 | 24 | 0.466 | 44 | 0.880 | 64 | 1.294 | 84 | 1.708 | A4 | 2.122 | C4 | 2.536 | E4 | 2.951 |
| 05 | 0.065 | 25 | 0.479 | 45 | 0.893 | 65 | 1.307 | 85 | 1.721 | A5 | 2.135 | C5 | 2.549 | E5 | 2.964 |
| 06 | 0.078 | 26 | 0.492 | 46 | 0.906 | 66 | 1.320 | 86 | 1.734 | A6 | 2.148 | C6 | 2.562 | E6 | 2.976 |
| 07 | 0.091 | 27 | 0.505 | 47 | 0.919 | 67 | 1.333 | 87 | 1.747 | A7 | 2.161 | C7 | 2.575 | E7 | 2.989 |
| 08 | 0.104 | 28 | 0.518 | 48 | 0.932 | 68 | 1.346 | 88 | 1.760 | A8 | 2.174 | C8 | 2.588 | E8 | 3.002 |
| 09 | 0.116 | 29 | 0.531 | 49 | 0.945 | 69 | 1.359 | 89 | 1.773 | A9 | 2.187 | C9 | 2.601 | E9 | 3.015 |
| 0A | 0.129 | 2A | 0.544 | 4A | 0.958 | 6A | 1.372 | 8A | 1.786 | AA | 2.200 | CA | 2.614 | EA | 3.028 |
| 0B | 0.142 | 2B | 0.556 | 4B | 0.971 | 6B | 1.385 | 8B | 1.799 | AB | 2.213 | CB | 2.627 | EB | 3.041 |
| 0C | 0.155 | 2C | 0.569 | 4C | 0.984 | 6C | 1.398 | 8C | 1.812 | AC | 2.226 | CC | 2.640 | EC | 3.054 |
| 0D | 0.168 | 2D | 0.582 | 4D | 0.996 | 6D | 1.411 | 8D | 1.825 | AD | 2.239 | CD | 2.653 | ED | 3.067 |
| 0E | 0.181 | 2E | 0.595 | 4E | 1.009 | 6E | 1.424 | 8E | 1.838 | AE | 2.252 | CE | 2.666 | EE | 3.080 |
| 0F | 0.194 | 2F | 0.608 | 4F | 1.022 | 6F | 1.436 | 8F | 1.851 | AF | 2.265 | CF | 2.679 | EF | 3.093 |
| 10 | 0.207 | 30 | 0.621 | 50 | 1.035 | 70 | 1.449 | 90 | 1.864 | B0 | 2.278 | D0 | 2.692 | F0 | 3.106 |
| 11 | 0.220 | 31 | 0.634 | 51 | 1.048 | 71 | 1.462 | 91 | 1.876 | B1 | 2.291 | D1 | 2.705 | F1 | 3.119 |
| 12 | 0.233 | 32 | 0.647 | 52 | 1.061 | 72 | 1.475 | 92 | 1.889 | B2 | 2.304 | D2 | 2.718 | F2 | 3.132 |
| 13 | 0.246 | 33 | 0.660 | 53 | 1.074 | 73 | 1.488 | 93 | 1.902 | B3 | 2.316 | D3 | 2.731 | F3 | 3.145 |
| 14 | 0.259 | 34 | 0.673 | 54 | 1.087 | 74 | 1.501 | 94 | 1.915 | B4 | 2.329 | D4 | 2.744 | F4 | 3.158 |
| 15 | 0.272 | 35 | 0.686 | 55 | 1.100 | 75 | 1.514 | 95 | 1.928 | B5 | 2.342 | D5 | 2.756 | F5 | 3.171 |
| 16 | 0.285 | 36 | 0.699 | 56 | 1.113 | 76 | 1.527 | 96 | 1.941 | B6 | 2.355 | D6 | 2.769 | F6 | 3.184 |
| 17 | 0.298 | 37 | 0.712 | 57 | 1.126 | 77 | 1.540 | 97 | 1.954 | B7 | 2.368 | D7 | 2.782 | F7 | 3.196 |
| 18 | 0.311 | 38 | 0.725 | 58 | 1.139 | 78 | 1.553 | 98 | 1.967 | B8 | 2.381 | D8 | 2.795 | F8 | 3.209 |
| 19 | 0.324 | 39 | 0.738 | 59 | 1.152 | 79 | 1.566 | 99 | 1.980 | B9 | 2.394 | D9 | 2.808 | F9 | 3.222 |
| 1A | 0.336 | 3A | 0.751 | 5A | 1.165 | 7A | 1.579 | 9A | 1.993 | BA | 2.407 | DA | 2.821 | FA | 3.235 |
| 1B | 0.349 | 3B | 0.764 | 5B | 1.178 | 7B | 1.592 | 9B | 2.006 | BB | 2.420 | DB | 2.834 | FB | 3.248 |
| 1C | 0.362 | 3C | 0.776 | 5C | 1.191 | 7C | 1.605 | 9C | 2.019 | BC | 2.433 | DC | 2.847 | FC | 3.261 |
| 1D | 0.375 | 3D | 0.789 | 5D | 1.204 | 7D | 1.618 | 9D | 2.032 | BD | 2.446 | DD | 2.860 | FD | 3.274 |
| 1E | 0.388 | 3E | 0.802 | 5E | 1.216 | 7E | 1.631 | 9E | 2.045 | BE | 2.459 | DE | 2.873 | FE | 3.287 |
| 1F | 0.401 | 3F | 0.815 | 5F | 1.229 | 7F | 1.644 | 9F | 2.058 | BF | 2.472 | DF | 2.886 | FF | 3.300 |

71

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.15.2 u2aEasyScale_Write

int u2aEasyScale_Write (U2A_HANDLE handle, BYTE DataBytes[ ], UInt16 nBytes,

BYTE *DataBits, UInt16 nBits, UInt16 WriteSpeed,

UInt16 WriteACK)

int CU2AClass::EasyScale_Write (BYTE DataBytes[ ], UInt16 nBytes, BYTE *DataBits, UInt16 nBits, UInt16 WriteSpeed, UInt16 WriteACK)

This function is used to write data to an EasyScaleTM device.
Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| DataBytes | An array of data bytes to be sent. |
| nBytes | The number of bytes in the DataBytes array. |
| DataBits | An array of additional bits to send. |
| nBits | The number of additional bits in the DataBits array. |
| WriteSpeed | The bit rate at which to send the data. See table in the Comments section. |
| WriteACK | Set to 1 to check for an ACK from the device, else set to 0. |

Comments:

The DataBytes parameter is a pointer to an array of bytes, with each element containing eight (8) bits of data to be sent. If the total number of bits to be sent is not an even multiple of eight, the remaining bits are in an array of bytes pointed to by the DataBits parameter.

The DataBits array may contain up to seven (7) elements, with each element representing one bit. If an element contains a zero (0) value, the bit value is interpreted to be zero (0). Any non-zero value (1 – 255) is construed to represent a bit value of one (1).

The total number of data bits sent to the device is (DataBytes * 8) + DataBits. Because of a packet size limitation and packet header overhead, the maximum number of bits that can be written is 408.

The WriteSpeed parameter must be one of the values from the following table:

| WriteSpeed | Bit rate |
| --- | --- |
| 0 | 10 kbps |
| 1 | 50 kbps |
| 2 | 100 kbps |
| 3 | 200 kbps |
| 4 | 400 kbps |

72

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.15.3 u2aEasyScale_Read

int u2aEasyScale_Read (U2A_HANDLE handle, UInt16 ReadSpeed, BYTE nBytes, BYTE nBits, BYTE DataBuffer[ ])

int CU2AClass::EasyScale_Read (UInt16 ReadSpeed, BYTE nBytes, BYTE nBits, BYTE DataBuffer[ ])

This function is used to read data from an EasyScaleTM device. Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| ReadSpeed | The bit rate at which to read the data. See table in the Comments section. |
| nBytes | The number of full 8-bit bytes to be read. |
| nBits | The number of additional bits to be read. |
| DataBuffer | An array to receive the data read. See Comments section for size requirement. |

Comments:

The DataBuffer parameter is a pointer to an array of bytes, which must be large enough to receive all of the data, plus some overhead data. The size of the array must be at least nBytes + 3 bytes.

Because of a packet size limitation and packet header overhead, the maximum number of bits that can be read is 408.

The ReadSpeed parameter must be one of the values from the following table:

| ReadSpeed | Bit rate |
| --- | --- |
| 0 | 10 kbps |
| 1 | 50 kbps |
| 2 | 100 kbps |
| 3 | 200 kbps |
| 4 | 400 kbps |

Return:

73

Returns zero on success. If an error occurs, a negative error code is returned.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.15.4 u2aEasyScale_WriteAndRead

int u2aEasyScale_WriteAndRead (U2A_HANDLE handle, UInt16 nBits,

BYTE WriteData[ ], BYTE ReadData[ ], BYTE WriteSpeed,

BYTE WriteACK)

int CU2AClass::EasyScale_WriteAndRead (UInt16 nBits, BYTE WriteData[ ], BYTE ReadData[], BYTE WriteSpeed, BYTE WriteACK)

This function is used to simultaneously write data to, and read data from, an EasyScaleTM device. Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| nBits | The total number of bits to be written and read. (See Comments section) |
| WriteData | An array of the data bytes to be sent. |
| ReadData | An array to receive the data bytes that are read. |
| WriteSpeed | The bit rate at which to send the data. See table in the Comments section. |
| WriteACK | Set to 1 to check for an ACK from the device, else set to 0. |

Comments:

The WriteData parameter is a pointer to an array of bytes, with each byte containing eight (8) bits of data to be sent. If the total number of bits to be sent (nBits) is not an even multiple of eight, the upper bits of the last byte will contain the remaining bits (any unnecessary lower bits are

ignored). For example, if 12 bits of data (010101100111) are to be sent, two bytes would be required (X represents ignored values):

56 7X (hex) or 01010110 0111XXXX (binary)

The size of the WriteData and ReadData arrays must be large enough to hold the data to be written and read, respectively. The size (in bytes) can be calculated as follows:

array size = (int)((nBits + 7) / 8)

Because of a packet size limitation and packet header overhead, the maximum allowable value of the nBits parameter is 408.

The WriteSpeed parameter must be one of the values from the following table:

| WriteSpeed | Bit rate |
| --- | --- |
| 0 | 10 kbps |
| 1 | 50 kbps |
| 2 | 100 kbps |

Return:

Returns the number of bits read on success. If an error occurs, a negative error code is returned.

74

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.16	DisplayScaleTM Functions

5.16.1 u2aDisplayScale_Setup

int u2aDisplayScale_Setup (U2A_HANDLE handle, UInt16 options, UInt16 speed) int CU2AClass:: DisplayScale_Setup (UInt16 options, UInt16 speed)

This function is used to set the parameters for DisplayScaleTM communications when using the u2aDisplayScale_xxxx I/O functions.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| options | Set to zero (0) to use the default behavior: write on GPIO0 and read on the rising edge of GPIO1. The behavior may be modified by adding one or more of the following values: 1 = Read on falling edge, instead of the rising edge 2 = Use GPIO7 for writing and GPIO6 for reading |
| speed | The bit rate used for communications, in kHz. Valid values are 15, 50, and 100. |

Comments:

There are currently no DisplayScale options, so the options parameter is ignored.

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.16.2 u2aDisplayScale_WriteReg

int u2aDisplayScale_WriteReg (U2A_HANDLE handle, BYTE address, BYTE Data)

int CU2AClass:: DisplayScale_WriteReg (BYTE address, BYTE Data)

75

This function is used to write a byte of data to a DisplayScaleTM register.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| address | 7-bit register address |
| Data | A byte of data to be written to the register. |

Comments:

The u2aDisplayScale_Setup function must be called at least once, to set the communications

parameters, before calling this function.

Data is sent with the bits transmitted from the most-significant to the least-significant, for each byte.

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.16.3 u2aDisplayScale_ReadReg

int u2aDisplayScale_ReadReg(U2A_HANDLE handle, BYTE address)

int CU2AClass:: DisplayScale_ReadReg(BYTE address)

This function is used to read a byte of data from a DisplayScaleTM register.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| address | 7-bit register address |

Comments:

The u2aDisplayScale_Setup function must be called at least once, to set the communications

parameters, before calling this function.

Data is read with the bits received from the most-significant to the least-significant, for each byte.

Return:

76

Returns the byte read on success. If an error occurs, a negative error code is returned.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.16.4 u2aDisplayScale_WriteAndRead

int u2aDisplayScale_WriteAndRead (U2A_HANDLE handle, UInt16 nBits, BYTE WriteData[ ], BYTE ReadData[ ])

int CU2AClass::DisplayScale_WriteAndRead (UInt16 nBits, BYTE WriteData[ ], BYTE ReadData[ ])

This function is used to simultaneously write data to, and read data from, a DisplayScaleTM device. Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| nBits | The total number of bits to be written and read. (See Comments section) |
| WriteData | An array of the data bytes to be sent. |
| ReadData | An array to receive the data bytes that are read. |

Comments:

The WriteData parameter is a pointer to an array of bytes, with each byte containing eight (8) bits of data to be sent. If the total number of bits to be sent (nBits) is not an even multiple of eight, the upper bits of the last byte will contain the remaining bits (any unnecessary lower bits are

ignored). For example, if 12 bits of data (010101100111) are to be sent, two bytes would be required (X represents ignored values):

56 7X (hex) or 01010110 0111XXXX (binary)

The size of the WriteData and ReadData arrays must be large enough to hold the data to be written and read, respectively. The size (in bytes) can be calculated as follows:

array size = (int)((nBits + 7) / 8)

Because of a packet size limitation and packet header overhead, the maximum allowable value of the nBits parameter is 408.

Return:

77

Returns the number of bits read on success. If an error occurs, a negative error code is returned.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.17	OneWire Functions

5.17.1 u2aOneWire_SetMode

int u2aOneWire_SetMode (U2A_HANDLE handle, UInt16 mode)

int CU2AClass::OneWire_SetMode (UInt16 mode)

This function sets the OneWire interface protocol mode or disables it.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| mode | Sets the OneWire interface mode. |

Comments:

The following table shows the valid values for the mode parameter:

| Constant Name | Value | Protocol Description | Output Pin |
| --- | --- | --- | --- |
| OW_Disable | 0 | Disables the OneWire interface. | N/A |
| OW_Mode1 | 1 | Enables Pulse Mode. | OW1 |
| OW_Mode2 | 2 | Enables Pulse Mode. | OW2 |
| OW_Mode3 | 3 | Enables Pulse Mode with Address. | OW3 |
| OW_Mode4 | 4 | Enables Pulse Mode. | OW4 |
| OW_Mode5 | 5 | Enables Pulse Mode with Address and extended (x10) pulse time. | OW5 |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.17.2 u2aOneWire_PulseSetup

int u2aOneWire_PulseSetup (U2A_HANDLE handle, UInt16 timeSetup,

UInt16 timeLow, UInt16 timeHigh, UInt16 timeStore,

int flags)

int CU2AClass::OneWire_PulseSetup (UInt16 timeSetup, UInt16 timeLow, UInt16 timeHigh, UInt16 timeStore, int flags)

This function sets the timing constants used by OneWire Pulse Mode on the currently enabled OWx pin.

78

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |  |  |  |
| --- | --- | --- | --- | --- |
| timeSetup | Defines the setup time, in microseconds. The valid range is 100 µs to 65535 µs. The default timeSetup value is 0 microseconds*. |  |  |  |
| timeLow | Defines the low pulse period, in microseconds. The valid range is 5µs to 65535µs. The default timeLow value is 10 µs*. If the OneWire interface is currently enabled in Mode 3, this value also sets the low pulse period for the address pulses. |  |  |  |
| timeHigh | Defines the high pulse period, in microseconds. The valid range is 5µs to 65535µs. The default timeHigh value is 10 µs*. If the OneWire interface is currently enabled in Mode 3, this value also sets the high pulse period for the address pulses. |  |  |  |
| timeStore | Defines the store time, in microseconds. The valid range is 100 µs to 65535 µs. The default timeStore value is 0 microseconds*. |  |  |  |
| flags | May be one or more (combined with logical OR) of the following values: |  |  |  |
|  | Flag Name | Value | Description |  |
|  | OW_FLAG_NONE* | 0 | Standard signal, as shown in Comments section. |  |
|  | OW_FLAG_INVERT | 1 | Invert all signal states. |  |
|  | OW_FLAG_NO_INIT | 2 | Suppress automatic setting of the signal’s initial and final states. |  |
|  | *The default is OW_FLAG_NONE, a non-inverted signal with automatic setting of the signal’s initial and final states. |  |  |  |

* The default setting will be in effect on the currently enabled OWx pin after the USB2ANY is reset and until u2aOneWire_PulseSetup is called to change the parameters.

Comments:

The following diagram illustrates how the timing constants affect the output signal.

Note: The resolution of all timers used for OneWire is 5µs. I.e., the time value specified for each parameter will be rounded down to the nearest multiple of 5µs. For example, whether you specify 10, 11, or 14 for the timeLow parameter, it will end up being set to 10µs.

The timeSetup and timeStore intervals of the output signal are optional and will be present only if the timeSetup and/or timeStore parameters are set to a value greater than zero.

In Mode 5, the actual time for all timing variables is the time specified multiplied by 10.

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

79

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.17.3 u2aOneWire_PulseWrite

int u2aOneWire_PulseWrite (U2A_HANDLE handle, Byte address, Byte pulses) int CU2AClass::OneWire_PulseWrite (Byte address, Byte pulses)

This function outputs the specified number of pulses on the currently selected OneWire interface output pin.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| address | Specifies the address for Mode 3 protocol. Ignored by other protocols. The valid range is 1 to 15. |
| pulses | Specifies the number of pulses to output. The valid range is 1 to 255. |

Comments:

This function is very similar to the u2aOneWire_PulseWriteEx function, except that the range of the address and pulses parameters is limited. If a larger value is needed for the address or pulses parameters, use the u2aOneWire_PulseWriteEx function instead of this one.

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.17.4 u2aOneWire_PulseWriteEx

int u2aOneWire_PulseWriteEx (U2A_HANDLE handle, Byte address, Uint16 pulses) int CU2AClass::OneWire_PulseWriteEx (Byte address, UInt16 pulses)

This function outputs the specified number of pulses on the currently selected OneWire interface output pin.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| address | Specifies the address for Mode 3 protocol. Ignored by other protocols. The valid range is 1 to 255. |
| pulses | Specifies the number of pulses to output. The valid range is 1 to 511. |

Comments:

This function is very similar to the u2aOneWire_PulseWrite function, except that the range of

the address and pulses parameters is extended.

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

80

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.17.5 u2aOneWire_SetOutput

int u2aOneWire_SetOutput (U2A_HANDLE handle, Byte state)

int CU2AClass::OneWire_SetOutput (Byte state)

This function sets the state of the output signal of the currently selected OneWire interface output pin.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| state | The desired state of the output. May be 0 (low) or 1 (high). |

Return:

81

Returns zero on success. If an error occurs, a negative error code is returned.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.18	Power Functions

5.18.1 u2aPower_WriteControl (Deprecated)

int u2aPower_WriteControl (U2A_HANDLE handle, Power_3V3 Power_3V3, Power_5V0 Power_5V0)

Warning: This function is deprecated. Use u2aPower_Enable for all new development.

This function enables/disables the 3.3V and 5.0V power outputs.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |  |  |
| --- | --- | --- | --- |
| Power_3V3 | Sets the state of the +3.3V_EXT power output pin. Valid values are: |  |  |
|  | Power_3V3_OFF | = | 0 |
|  | Power_3V3_ON | = | 1 |
| Power_5V0 | Sets the state of the +5V_EXT power output pin. Valid values are: |  |  |
|  | Power_5V0_OFF | = | 0 |
|  | Power_5V0_ON | = | 1 |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.18.2 u2aPower_Enable

int u2aPower_Enable (U2A_HANDLE handle, int Enable3V3, int Enable5v0, int EnableAdj)

82

int CU2AClass::Power_Enable (int Enable3V3, int Enable5v0, int EnableAdj) This function enables/disables the 3.3V, 5.0V, and Adjustable power outputs.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |  |  |
| --- | --- | --- | --- |
| Enable3V3 | Sets the state of the +3.3V_EXT power output pin. Valid values are: |  |  |
|  | Power_3V3_Disable | = | 0 |
|  | Power_3V3_Enable | = | 1 |
|  | Power_3V3_Ignore | = | 2 |
| Enable5v0 | Sets the state of the +5V_EXT power output pin. Valid values are: |  |  |
|  | Power_5V0_Disable | = | 0 |
|  | Power_5V0_Enable | = | 1 |
|  | Power_5V0_Ignore | = | 2 |
| EnableAdj | ONEDEMO Sets the state of the ADJ_2.5-5V_DUT | power output pin. Valid |  |
|  | values are: |  |  |
|  | Power_ADJ_Disable | = | 0 |
|  | Power_ADJ_Enable | = | 1 |
|  | Power_ADJ_Ignore | = | 2 |

Comments:

For each output, Power_xxx_Disable turns the power OFF, Power_xxx_Enable turns it ON, and

Power_xxx_Ignore leaves it in its current state.

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.18.3 u2aPower_SetVoltageRef

int u2aPower_SetVoltageRef (U2A_HANDLE handle, int Value) int CU2AClass::Power_SetVoltageRef (int Value)

This function sets the output voltage of the adjustable power supply. Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| Value | The 8-bit DAC reference value used to set the output voltage. |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.18.4 u2aPower_ReadStatus

int u2aPower_ReadStatus (U2A_HANDLE handle)
int CU2AClass::Power_ReadStatus ()

This function reads the status of the 3.3V and 5.0V power outputs.

83

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |

Return:

On success, returns one of the values from the table below. If an error occurs, a negative error

code is returned.

Return values for USB2ANY:

| Return Value | 3.3V Power | 5.0V Power |
| --- | --- | --- |
| 0 | OK | OK |
| 1 | FAULT | OK |
| 2 | OK | FAULT |
| 3 | FAULT | FAULT |

Return values for OneDemo:

84

| Return Value | 3.3V Power | 5.0V Power | Adjustable |
| --- | --- | --- | --- |
| 0 | OK | OK | OK |
| 1 | FAULT | OK | OK |
| 2 | OK | FAULT | OK |
| 3 | FAULT | FAULT | OK |
| 4 | OK | OK | FAULT |
| 5 | FAULT | OK | FAULT |
| 6 | OK | FAULT | FAULT |
| 7 | FAULT | FAULT | FAULT |

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.19	Miscellaneous Functions

5.19.1 u2aFirmwareVersion_Read

int u2aFirmwareVersion_Read(U2A_HANDLE handle, BYTE *pBuffer, int nBufferSize) int CU2AClass::FirmwareVersion_Read(BYTE *pBuffer, int nBufferSize)

This function reads the firmware version number of the USB2ANY controller associated with the specified handle.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| pBuffer | Pointer to a buffer, which will receive the version data. The buffer must be large enough to receive at least four (4) bytes of data. |
| dwBufferSize | The size of the buffer (in bytes), pointed to by pBuffer. |

Return:

Returns the number of bytes received and copied to the provided buffer. If an error occurs, a negative error code is returned.

The format of the data returned in the buffer pointed to by pBuffer is as follows:

| Byte Offset | Description |
| --- | --- |
| 0 | Major version |
| 1 | Minor version |
| 2 | Major revision |
| 3 | Minor revision |

5.19.2 u2aLED_WriteControl (USB2ANY)

int u2aLED_WriteControl (U2A_HANDLE handle, LED LEDState)

int CU2AClass::LED_WriteControl (LED LEDState)

85

This function is used to control the LED on the USB2ANY.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |  |
| --- | --- | --- |
| LEDState | Sets the state of the on-board LED. Valid values are: |  |
|  | LED_OFF = | 0 |
|  | LED_ON = | 1 |
|  | LED_TOGGLE = | 2 |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

5.19.3 u2aLED_SetState (OneDemo)

int u2aLED_SetState (U2A_HANDLE handle, int LEDState, int BlinkCode) int CU2AClass::LED_SetState (int LEDState, int BlinkCode)

This function is used to control the green LED on the USB2ANY or the bi-color (red/green) LED on the OneDemo.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| LEDState | Sets the state of the on-board LEDs. Valid value is one or more of: LEDS_OFF = 0 LEDS_RED_ON = 1 (ignored by USB2ANY) LEDS_GREEN_ON = 2 LEDS_TOGGLE = 4 |
| BlinkCode | Sets the blink pattern of the green LED or bi-color LED. See the table in the Comments section for descriptions of the available blink patterns. |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

Comments:

This function can used to set the visible color of the LED to red, green, or yellow (both LEDs on).

If the LEDS_TOGGLE bit is set, the current state of the LED(s) is toggled and the other bits of the parameter are ignored.

Note that, when BlinkCode is set non-zero on the OneDemo, the LEDs blink simultaneously, in the color defined by LEDState (except for LEDS_BLINK_ALT – see description below). The blink patterns are defined by the descriptions in the following table:

| Symbolic Name | Value | Description of repeating blink pattern |
| --- | --- | --- |
| LEDS_NON_BLINK | 0 | When on, LEDs will not blink. |

86

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

| Symbolic Name | Value | Description of repeating blink pattern |
| --- | --- | --- |
| LEDS_BLINK_1 | 1 | BLINK pause |
| LEDS_BLINK_2 | 2 | BLINK BLINK pause |
| LEDS_BLINK_3 | 3 | BLINK BLINK BLINK pause |
| LEDS_BLINK_4 | 4 | BLINK BLINK BLINK BLINK pause |
| LEDS_BLINK_FAST | 5 | Fast blinking, with no pause |
| LEDS_BLINK_ALT | 6 | OneDemo: Red and green LEDs blink alternately USB2ANY: Slow blinking, with no pause |

When a non-zero BlinkCode is specified, the LED(s) will blink in the color specified by the LEDState parameter. For LEDS_BLINK_ALT to work properly on the OneDemo, both red and green LEDs must be turned on.

5.19.4 u2aClock_Control

int u2aClock_Control (U2A_HANDLE handle, ClockDivider1 ClockDivider1, ClockDivider2 ClockDivider2)

int CU2AClass::Clock_Control (ClockDivider1 ClockDivider1, ClockDivider2 ClockDivider2)

This function sets the clock divider.
Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |  |  |
| --- | --- | --- | --- |
| ClockDivider1 | Sets the first clock divider. Must be one of the following values: |  |  |
|  | ClockDiv1_1 | = | 0 |
|  | ClockDiv1_2 | = | 1 |
|  | ClockDiv1_4 | = | 2 |
|  | ClockDiv1_8 | = | 3 |
|  | ClockDiv1_16 | = | 4 |
|  | ClockDiv1_32 | = | 5 |
| ClockDivider2 | Sets the second clock divider. Must be one of the following values: |  |  |
|  | ClockDiv2_1 | = | 0 |
|  | ClockDiv2_2 | = | 1 |
|  | ClockDiv2_4 | = | 2 |
|  | ClockDiv2_8 | = | 3 |
|  | ClockDiv2_16 | = | 4 |
|  | ClockDiv2_32 | = | 5 |

Return:

Returns zero on success. If an error occurs, a negative error code is returned.

87

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.20	Status Functions

5.20.1 u2aStatus_IsUSB2ANYConnected

int u2aStatus_IsUSB2ANYConnected ()

int CU2AClass::Status_IsUSB2ANYConnected ()

This function reports whether the USB2ANY (or OneDemo) is connected.

Parameters:
None

Comments:

The return value of this function is meaningful only after a successful call to the u2aEnableDeviceDetect function. Otherwise, the connection status cannot be determined and the return value is always FALSE.

Return:

Returns TRUE (1) if a USB2ANY or OneDemo device is connected, otherwise FALSE (0).

5.20.2 u2aStatus_GetErrorCode

int u2aStatus_GetErrorCode (U2A_HANDLE handle)
int CU2AClass::Status_GetErrorCode ()

This function retrieves the most recent status/error code for the device specified by handle.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |

Comments:

The u2aStatus_GetErrorCode function returns error status for the last API function executed. Therefore, it is recommended that the function be called immediately after the API function for which the error status is desired. The previous error status is cleared before every API function is executed and new error status is stored only until the next API function is executed.

This is a “one-shot” function that returns error codes only once for the most recently executed

function. Any reported error code is cleared immediately after this function executes. The returned error status must be stored locally if it will be used more than once.

88

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

If this function is called more than once between API commands, only the first call will return an error code and all subsequent calls will return zero (i.e., no error).

Return:

Returns the most recent status/error code for last API function called on the specified device.

5.20.3 u2aStatus_GetText

char * u2aStatus_GetText (int Code, char *TextBuffer, UInt16 nBufferSize) char * CU2AClass::Status_GetText (int Code, char *TextBuffer, UInt16 nBufferSize)

This function retrieves readable English text for a given status/error code.

Parameters:

| Code | The status/error code for which to obtain readable text. |
| --- | --- |
| TextBuffer | Pointer to a buffer to receive the null-terminated text. |
| nBufferSize | The size of the buffer to receive the text. |

Return:

Returns a pointer to TextBuffer, which will contain the text for the specified code. If TextBuffer is too small to hold the entire text, the text will be truncated. A buffer size of at least 40 bytes (i.e., nBufferSize >= 40) is recommended.

5.20.4 u2aStatus_GetControllerType

int u2aStatus_GetControllerType (U2A_HANDLE handle)
int CU2AClass::Status_GetControllerType ()

This function retrieves the controller type opened on handle.

Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |

Return:

Returns a value that designates the controller type:

| Returned Value | Controller Type |
| --- | --- |
| 0 | Unknown |
| 1 | USB2ANY |
| 2 | OneDemo |

89

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

5.20.5 u2aStatus_EVMDetect

int u2aStatus_EVMDetect (U2A_HANDLE handle, int nFunction, void *Callback) int CU2AClass::Status_EVMDetect (int nFunction, void *Callback)

This function controls the operation of the EVM DETECT pin.
Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |
| nFunction | Defines the function to be performed: EVM_DetectDisable = 0 EVM_DetectEnable = 1 EVM_DetectQuery = 2 |
| Callback | The address of the callback function to be called when an EVM board is connected or disconnected. If set to NULL, no function is called, but board connection/disconnection can be detected use a polled mode. See Comments section for more information. This parameter is ignored if nFunction is not set to 1 (EVM_DetectEnable). |

Comments:

The EVM DETECT signal shares a pin with INT0 signal. When EVM detection is enabled, INT0

will not be available for other purposes.

If Callback is set to a non-Null value, the function at the specified address will be called when an

EVM board is connected or disconnected. The callback function must have the following prototype:

void _stdcall CallBack(int channel, DWORD flag)

There will be two parameters passed to the function. The value of the first parameter is the interrupt identifier (always equal to 4). The value of the second parameter will be zero (0) if an EVM board was disconnected or one (1) if a board was connected.

If the use of a callback function is not desired, EVM board connection/disconnection can be detected using a polling method. To use the polling method, do the following:

- Enable EVM board detection in polled mode:

| u2aStatus _ | EVMDetect(handle, 1, NULL); |
| --- | --- |

- Call u2aStatus_EVM Detect with nFunction set to 2 and Callback set to NULL:

| Ret = u2aStatus _ | EVMDetect(handle, 2, NULL); |
| --- | --- |

- The function will return one (1) if a board is connected or zero (0) if not.

90

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Return:

The EVM_DetectQuery function returns zero if the EVM DETECT pin is low, one if it is high. On

success, the other functions return zero. If an error occurs, a negative error code is returned.

5.20.6 u2aStatus_GetBoardRevision

int u2aStatus_GetBoardRevision (U2A_HANDLE handle)
int CU2AClass::Status_GetBoardRevision ()

This function returns the hardware revision level of the OneDemo board that is currently open. Parameters:

| handle | A valid handle, obtained by a call to the u2aOpen function. |
| --- | --- |

Return:

On success, the PCB Assembly revision number is returned. The revision number is always a positive value, representing a revision letter (0=A, 1=B, 2=C, etc.). If an error occurs, a negative error code is returned.

The USB2ANY is not able to detect the board revision and always returns zero (0) in response to this function.

91

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Appendix A: Error Codes

Virtually all of the API functions return an integer value. In general, the return value represents an error code if it is a negative value. In most cases, a return value of zero means “Success” or “No error”. Some functions return the result of the function as a positive number.

See the Return: section of the each individual function description for specific details regarding the return value.

Error codes and descriptions by value

| Value | Mnemonic | Description |
| --- | --- | --- |
| 0 | ERR_OK | No error |
| -1 | ERR_COM_RX_OVERFLOW | Receiver overflowed |
| -2 | ERR_COM_RX_BUF_EMPTY | Receive buffer is empty |
| -3 | ERR_COM_TX_BUF_FULL | Transmit buffer is full |
| -4 | ERR_COM_TX_STALLED | Transmit is stalled |
| -5 | ERR_COM_TX_FAILED | Transmit failed |
| -6 | ERR_COM_OPEN_FAILED | Failed to open communications port |
| -7 | ERR_COM_PORT_NOT_OPEN | Communications port is not open |
| -8 | ERR_COM_PORT_IS_OPEN | Communications port is open |
| -9 | ERR_COM_READ_TIMEOUT | Receive timeout |
| -10 | ERR_COM_READ_ERROR | Communications port read error |
| -11 | ERR_COM_WRITE_ERROR | Communications port write error |
| -12 | ERR_DEVICE_NOT_FOUND | Communications device not found |
| -13 | ERR_COM_CRC_FAILED | Communications CRC failed |
| -20 | ERR_INVALID_PORT | Invalid port |
| -21 | ERR_ADDRESS_OUT_OF_RANGE | Address is out of accepted range |
| -22 | ERR_INVALID_FUNCTION_CODE | Invalid function code |
| -23 | ERR_BAD_PACKET_SIZE | Invalid packet size |
| -24 | ERR_INVALID_HANDLE | Invalid handle |
| -25 | ERR_OPERATION_FAILED | Operation failed |
| -26 | ERR_PARAM_OUT_OF_RANGE | Parameter is out of range |
| -27 | ERR_PACKET_OUT_OF_SEQUENCE | Packet is out of sequence |
| -28 | ERR_INVALID_PACKET_HEADER | Invalid packet header |
| -29 | ERR_UNIMPLEMENTED_FUNCTION | Function not implemented |
| -30 | ERR_TOO_MUCH_DATA | Too much data |
| -31 | ERR_INVALID_DEVICE | Invalid device |
| -32 | ERR_UNSUPPORTED_FIRMWARE | Unsupported firmware version |
| -33 | ERR_BUFFER_TOO_SMALL | Buffer is too small |
| -34 | ERR_NO_DATA | No data available |
| -35 | ERR_RESOURCE_CONFLICT | Resource conflict |
| -36 | ERR_NO_EVM | EVM is required for external power |
| -37 | ERR_COMMAND_BUSY | Command is busy |
| -38 | ERR_ADJ_POWER_FAIL | Adjustable power supply failure |
| -39 | ERR_NOT_ENABLED | Interface or mode is not enabled |
| -40 | ERR_I2C_INIT_ERROR | I2C initialization failed |
| -41 | ERR_I2C_READ_ERROR | I2C read error |
| -42 | ERR_I2C_WRITE_ERROR | I2C write error |
| -43 | ERR_I2C_BUSY | I2C busy (transfer is pending) |

92

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

93

| Value | Mnemonic | Description |
| --- | --- | --- |
| -44 | ERR_I2C_ADDR_NAK | Address not acknowledged (NAK) |
| -45 | ERR_I2C_DATA_NAK | Data not acknowledged (NAK) |
| -46 | ERR_I2C_READ_TIMEOUT | Read timeout |
| -47 | ERR_I2C_READ_DATA_TIMEOUT | Read data timeout |
| -48 | ERR_I2C_READ_COMP_TIMEOUT | Timeout waiting for read complete |
| -49 | ERR_I2C_WRITE_TIMEOUT | Write timeout |
| -50 | ERR_I2C_WRITE_DATA_TIMEOUT | Write data timeout |
| -51 | ERR_I2C_WRITE_COMP_TIMEOUT | Timeout waiting for write complete |
| -52 | ERR_I2C_NOT_MASTER | I2C not in Master mode |
| -53 | ERR_I2C_ARBITRATION_LOST | I2C arbitration lost |
| -54 | ERR_I2C_NO_PULLUP_POWER | I2C pullups require the 3.3V EXT power to be on |
| -60 | ERR_SPI_INIT_ERROR | SPI initialization failed |
| -61 | ERR_SPI_WRITE_READ_ERROR | SPI write/read error |
| -70 | ERR_DATA_WRITE_ERROR | Data write error |
| -71 | ERR_DATA_READ_ERROR | Data read error |
| -72 | ERR_TIMEOUT | Operation timeout |
| -73 | ERR_DATA_CRC_FAILED | Data CRC failed |

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Appendix B: Exported Symbols

USB2ANY.DLL is a 32-bit dynamic-linked library with all functions exported using the __stdcall calling convention. The 32-bit versions of Visual C++ define PASCAL as __stdcall, which may cause some confusion.

In older 16-bit versions of Visual C++, PASCAL names were simply undecorated function names in all uppercase letters. However, __stdcall in 32-bit versions of Visual C++ allow mixed-case names and decorate the name by prefixing it with an underscore (_) and appending an at-sign (@) followed by the required stack space (in bytes). For example, a function declared as:

int	stdcall MyFunc (int x, int y)

would be decorated as:

MyFunc@8

Note that __stdcall is like PASCAL in the sense that the stack is cleaned-up by the called function (in this case, by the USB2ANY function).

Exported Symbols (by API Function Name value)

| API Function Name | Ordinal* | Exported Name |
| --- | --- | --- |
| u2aADC_Control | 19 | _u2aADC_Control@24 |
| u2aADC_ConvertAndRead | 20 | _u2aADC_ConvertAndRead@12 |
| u2aClock_Control | 21 | _u2aClock_Control@12 |
| u2aClose | 22 | _u2aClose@4 |
| u2aDACs_Write | 23 | _u2aDACs_Write@16 |
| u2aEnableDebugLogging | 24 | _u2aEnableDebugLogging@4 |
| u2aEnableDeviceDetect | 25 | _u2aEnableDeviceDetect@8 |
| u2aFEC_Control | 26 | _u2aFEC_Control@56 |
| u2aFEC_CountAndRead | 27 | _u2aFEC_CountAndRead@4 |
| u2aFindControllers | 28 | _u2aFindControllers@0 |
| u2aFirmwareVersion_Read | 29 | _u2aFirmwareVersion_Read@12 |
| u2aGetErrorList | 36 | _u2aGetErrorList@12 |
| u2aGetSerialNumber | 37 | _u2aGetSerialNumber@8 |
| u2aGetSerialNumberW | 38 | _u2aGetSerialNumberW@8 |
| u2aGetStatusText | 39 | _u2aGetStatusText@12 |
| u2aGPIO_ReadPort | 30 | _u2aGPIO_ReadPort@8 |
| u2aGPIO_ReadState | 31 | _u2aGPIO_ReadState@12 |
| u2aGPIO_SetPort | 32 | _u2aGPIO_SetPort@12 |
| u2aGPIO_WriteControl | 33 | _u2aGPIO_WriteControl@56 |
| u2aGPIO_WritePort | 34 | _u2aGPIO_WritePort@12 |
| u2aGPIO_WriteState | 35 | _u2aGPIO_WriteState@56 |
| u2aI2C_Control | 40 | _u2aI2C_Control@16 |
| u2aI2C_InternalRead | 41 | _u2aI2C_InternalRead@24 |
| u2aI2C_InternalWrite | 42 | _u2aI2C_InternalWrite@24 |
| u2aI2C_MultiRegisterRead | 43 | _u2aI2C_MultiRegisterRead@20 |
| u2aI2C_MultiRegisterWrite | 44 | _u2aI2C_MultiRegisterWrite@20 |
| u2aI2C_RawRead | 45 | _u2aI2C_RawRead@16 |
| u2aI2C_RawWrite | 46 | _u2aI2C_RawWrite@16 |

94

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

| API Function Name | Ordinal* | Exported Name |
| --- | --- | --- |
| u2aI2C_RegisterRead | 47 | _u2aI2C_RegisterRead@12 |
| u2aI2C_RegisterWrite | 48 | _u2aI2C_RegisterWrite@16 |
| u2aInterrupt_Control | 49 | _u2aInterrupt_Control@20 |
| u2aIsUSB2ANYConnected | 50 | _u2aIsUSB2ANYConnected@0 |
| u2aLED_WriteControl | 51 | _u2aLED_WriteControl@8 |
| u2aMSP430_ByteRead | 52 | _u2aMSP430_ByteRead@8 |
| u2aMSP430_ByteWrite | 53 | _u2aMSP430_ByteWrite@12 |
| u2aMSP430_WordRead | 54 | _u2aMSP430_WordRead@8 |
| u2aMSP430_WordWrite | 55 | _u2aMSP430_WordWrite@12 |
| u2aOpen | 56 | _u2aOpen@4 |
| u2aOpenW | 57 | _u2aOpenW@4 |
| u2aPower_ReadStatus | 59 | _u2aPower_ReadStatus@4 |
| u2aPower_WriteControl | 60 | _u2aPower_WriteControl@12 |
| u2aPWM_Control | 58 | _u2aPWM_Control@32 |
| u2aReadResponse | 61 | _u2aReadResponse@12 |
| u2aReadResponseB64 | 62 | _u2aReadResponseB64@12 |
| u2aSetReceiveTimeout | 66 | _u2aSetReceiveTimeout@4 |
| u2aSetResponseMode | 67 | _u2aSetResponseMode@4 |
| u2aSPI_Control | 63 | _u2aSPI_Control@36 |
| u2aSPI_WriteAndRead | 64 | _u2aSPI_WriteAndRead@12 |
| u2aSPI_WriteAndRead_P | 65 | _u2aSPI_WriteAndRead_P@12 |
| u2aStatus_GetErrorCode | 68 | _u2aStatus_GetErrorCode@4 |
| u2aStatus_GetText | 69 | _u2aStatus_GetText@12 |
| u2aStatus_GetTextW | 70 | _u2aStatus_GetTextW@12 |
| u2aUART_Control | 71 | _u2aUART_Control@24 |
| u2aUART_Read | 72 | _u2aUART_Read@12 |
| u2aUART_Write | 73 | _u2aUART_Write@12 |

* Note: It is recommended that you use the exported name, instead of the ordinal, because the ordinals may change in subsequent releases of the API library.

95

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Appendix C: Visual Basic Interface (VB6)

'

' Declarations for USB2ANY.DLL subroutines and functions

Public Declare Function u2aFindControllers Lib "USB2ANY" Alias "_u2aFindControllers@0" () As Long

Public Declare Function u2aGetSerialNumber Lib "USB2ANY" Alias "_u2aGetSerialNumber@8" (ByVal index As Long, _

ByVal SerialNumber As String) As Long

Public Declare Function u2aOpen Lib "USB2ANY" Alias "_u2aOpen@4" (ByVal SerialNumber As String) As Long

Public Declare Function u2aGetStatusText Lib "USB2ANY" Alias "_u2aGetStatusText@12" (ByVal code As Long, _

ByVal buffer As String, ByVal bufsize As Long) As String

Public Declare Function u2aSetReceiveTimeout Lib "USB2ANY" Alias "_u2aSetReceiveTimeout@4" (ByVal milliseconds As Long) As Long

'/////////////////////////////////////////////////////////////////////////////////////////////////////////////

'//The following API functions require a handle parameter. They MUST be called with a valid

'//handle, obtained from a successful call to the u2aOpen() function.

'/////////////////////////////////////////////////////////////////////////////////////////////////////////////

'//

'// Controller Functions

'//

Public Declare Function u2aClose Lib "USB2ANY" Alias "_u2aClose @4" (ByVal handle As Long) As Long

Public Declare Function u2aReadResponse Lib "USB2ANY" Alias "_u2aReadResponse@12" (ByVal handle As Long, ByVal pBuffer As String, _

ByVal dwBufferSize As Long) As Long

'//

'// I2C Functions

'//

Public Declare Function u2aI2C_Control Lib "USB2ANY" Alias "_u2aI2C_Control@20" (ByVal handle As Long, ByVal Speed As Long, _

ByVal AddressLength As Long, ByVal PullUps As Long) As Long

Public Declare Function u2aI2C_RegisterRead Lib "USB2ANY" Alias "_u2aI2C_RegisterRead@12" (ByVal handle As Long, _

ByVal I2C_Address As Long, ByVal RegisterAddress As Long) As Long

Public Declare Function u2aI2C_RegisterWrite Lib "USB2ANY" Alias "_u2aI2C_RegisterWrite@16" (ByVal handle As Long, _

ByVal I2C_Address As Long, ByVal RegisterAddress As Long, ByVal Value As Long) As Long

Public Declare Function u2aI2C_MultiRegisterRead Lib "USB2ANY" Alias "_u2aI2C_MultiRegisterRead@20" (ByVal handle As Long, _

ByVal I2C_Address As Long, ByVal StartingRegisterAddress As Long, ByVal nBytes As Long, ByVal Data[] As Long) As Long

Public Declare Function u2aI2C_MultiRegisterWrite Lib "USB2ANY" Alias "_u2aI2C_MultiRegisterWrite@20" (ByVal handle As Long, _

ByVal I2C_Address As Long, ByVal StartingRegisterAddress As Long, ByVal nBytes As Long, ByVal Data[] As Long) As Long

Public Declare Function u2aI2C_InternalRead Lib "USB2ANY" Alias "_u2aI2C_InternalRead@24" (ByVal handle As Long, _

ByVal I2C_Slave_Address As Long, ByVal InternalAddress As Long, ByVal nIntBytes As Long, ByVal nBytes As Long, _

ByVal Data[] As Long) As Long

Public Declare Function u2aI2C_InternalWrite Lib "USB2ANY" Alias "_u2aI2C_InternalWrite@24" (ByVal handle As Long, _

ByVal I2C_Slave_Address As Long, ByVal InternalAddress As Long, ByVal nIntBytes As Long, ByVal nBytes As Long, _

ByVal Data[] As Long) As Long

Public Declare Function u2aI2C_RawRead Lib "USB2ANY" Alias "_u2aI2C_RawRead@16" (ByVal handle As Long, _

ByVal I2C_Address As Long, ByVal nBytes As Long, ByVal Data[] As Long) As Long

Public Declare Function u2aI2C_RawWrite Lib "USB2ANY" Alias "_u2aI2C_RawWrite@16" (ByVal handle As Long, ByVal I2C_Address As Long, _

ByVal nBytes As Long, ByVal Data[] As Long) As Long

'//

'// SPI Functions

'//

Public Declare Function u2aSPI_Control Lib "USB2ANY" Alias "_u2aSPI_Control@36" (ByVal handle As Long, _

ByVal _SPI_ClockPhase As Long, _

ByVal _SPI_ClockPolarity As Long, _

ByVal _SPI_BitDirection As Long, _

ByVal _SPI_CharacterLength As Long, _

ByVal _SPI_LatchType As Long, _

ByVal _SPI_LatchPolarity As Long, _

ByVal _DividerHigh As Long, _

ByVal _DividerLow As Long RPAREN As Long

Public Declare Function u2aSPI_WriteAndRead Lib "USB2ANY" Alias "_u2aSPI_WriteAndRead@12" (ByVal handle As Long, _

96

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

ByVal nBytes As Long, ByVal Data[] As Long) As Long

'//

'// ADC, DAC, and PWM Functions

'//

Public Declare Function u2aADC_Control Lib "USB2ANY" Alias "_u2aADC_Control@24" (ByVal handle As Long, _

ByVal ADC0 As Long, _

ByVal ADC1 As Long, _

ByVal ADC2 As Long, _

ByVal ADC3 As Long, _

ByVal VREF As Long) As Long

Public Declare Function u2aADC_ConvertAndRead Lib "USB2ANY" Alias "_u2aADC_ConvertAndRead@12" (ByVal handle As Long, _

ByVal nBytes As Long, ByVal Data[] As Long) As Long

Public Declare Function u2aDACs_Write Lib "USB2ANY" Alias "_u2aDACs_Write@20" (ByVal handle As Long, ByVal _DACs_WhichDAC As Long, _

ByVal _DACs_OperatingMode As Long, ByVal _Value As Long) As Long

Public Declare Function u2aPWM_Control Lib "USB2ANY" Alias "_u2aPWM_Control@32" (ByVal handle As Long, _

ByVal _ModeControl As Long, _

ByVal _PWM_WhichPWM As Long, _

ByVal _InputDivider As Long, _

ByVal _CompareRegister0 As Long, _

ByVal _OutputMode1 As Long, _

ByVal _CompareRegister1 As Long, _

ByVal _InputDividerEX As Long) As Long

'//

'// UART Functions

'//

Public Declare Function u2aUART_Control Lib "USB2ANY" Alias "_u2aUART_Control@24" (ByVal handle As Long, _

ByVal _UART_BaudRate As Long, _

ByVal _UART_Parity As Long, _

ByVal _UART_BitDirection As Long, _

ByVal _UART_CharacterLength As Long, _

ByVal _UART_StopBits As Long) As Long

Public Declare Function u2aUART_Write Lib "USB2ANY" Alias "_u2aUART_Write@12" (ByVal handle As Long , _

ByVal nBytes As Long, ByVal Data[] As Long) As Long

Public Declare Function u2aUART_Read Lib "USB2ANY" Alias "_u2aUART_Read@12" (ByVal handle As Long, _

ByVal nBytes As Long, ByVal Data[] As Long) As Long

'//

'// GPIO Functions

'//

Public Declare Function u2aGPIO_WriteControl Lib "USB2ANY" Alias "_u2aGPIO_WriteControl@56" (ByVal handle As Long, _

ByVal GPIO0 As Long, _
ByVal GPIO1 As Long, _
ByVal GPIO2 As Long, _
ByVal GPIO3 As Long, _
ByVal GPIO4 As Long, _
ByVal GPIO5 As Long, _
ByVal GPIO6 As Long, _
ByVal GPIO7 As Long, _
ByVal GPIO8 As Long, _
ByVal GPIO9 As Long, _
ByVal GPIO10 As Long, _
ByVal GPIO11 As Long, _
ByVal GPIO12 As Long) As Long

Public Declare Function u2aGPIO_WriteState Lib "USB2ANY" Alias "_u2aGPIO_WriteState@56" (ByVal handle As Long, _

ByVal GPIO0 As Long, _ ByVal GPIO1 As Long, _ ByVal GPIO2 As Long, _ ByVal GPIO3 As Long, _ ByVal GPIO4 As Long, _ ByVal GPIO5 As Long, _ ByVal GPIO6 As Long, _ ByVal GPIO7 As Long, _ ByVal GPIO8 As Long, _ ByVal GPIO9 As Long, _ ByVal GPIO10 As Long, _ ByVal GPIO11 As Long, _

97

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

ByVal GPIO12 As Long) As Long

Public Declare Function u2aGPIO_ReadState Lib "USB2ANY" Alias "_u2aGPIO_ReadState@12" (ByVal handle As Long , _

ByVal nBytes As Long, ByVal Data[] As Long) As Long

Public Declare Function u2aGPIO_SetPort Lib "USB2ANY" Alias "_u2aGPIO_SetPort@12" (ByVal handle As Long, _

ByVal GPIO_Port As Long, ByVal function As Long) As Long

Public Declare Function u2aGPIO_WritePort Lib "USB2ANY" Alias "_u2aGPIO_WritePort@12" (ByVal handle As Long, _

ByVal GPIO_Port As Long, ByVal state As Long) As Long

Public Declare Function u2aGPIO_ReadPort Lib "USB2ANY" Alias "_u2aGPIO_ReadPort@8" (ByVal handle As Long, _

ByVal GPIO_Port As Long) As Long

'//

'// Memory Functions

'//

Public Declare Function u2aMSP430_ByteRead Lib "USB2ANY" Alias "_u2aMSP430_ByteRead@8" (ByVal handle As Long , _

ByVal Address As Long) As Long

Public Declare Function u2aMSP430_ByteWrite Lib "USB2ANY" Alias "_u2aMSP430_ByteWrite@12" (ByVal handle As Long, _

ByVal Address As Long, ByVal Value As Long) As Long

Public Declare Function u2aMSP430_WordRead Lib "USB2ANY" Alias "_u2aMSP430_WordRead@8" (ByVal handle As Long, _

ByVal Address As Long) As Long

Public Declare Function u2aMSP430_WordWrite Lib "USB2ANY" Alias "_u2aMSP430_WordWrite@12" (ByVal handle As Long, _

ByVal Address As Long, ByVal Value As Long) As Long

Public Declare Function u2aMSP430_MemoryRead Lib "USB2ANY" Alias "_u2aMSP430_MemoryRead@16" (ByVal handle As Long, _

ByVal Address As Long, ByVal nBytes As Long, ByVal *Data As Long) As Long

Public Declare Function u2aMSP430_MemoryWrite Lib "USB2ANY" Alias "_u2aMSP430_MemoryWrite@16" (ByVal handle As Long, _

ByVal Address As Long, ByVal nBytes As Long, ByVal *Data As Long) As Long

'//

'// Miscellaneous Functions

'//

Public Declare Function u2aFirmwareVersion_Read Lib "USB2ANY" Alias "_u2aFirmwareVersion_Read@12" (ByVal handle As Long , _

ByVal szVersion As String, ByVal bufsize As Long) As Long

Public Declare Function u2aPower_WriteControl Lib "USB2ANY" Alias "_u2aPower_WriteControl@12" (ByVal handle As Long, _

ByVal _Power_3V3 As Long, ByVal _Power_5V0 As Long) As Long

Public Declare Function u2aPower_ReadStatus Lib "USB2ANY" Alias "_u2aPower_ReadStatus@4" (ByVal handle As Long) As Long

Public Declare Function u2aLED_WriteControl Lib "USB2ANY" Alias "_u2aLED_WriteControl@8" (ByVal handle As Long , _

ByVal _LED As Long) As Long

Public Declare Function u2aClock_Control Lib "USB2ANY" Alias "_u2aClock_Control@12" (ByVal handle As Long, _

98

ByVal _ClockDivider1 As Long, ByVal _ClockDivider2 As Long) As Long

API Reference for the USB2ANY API Specification

99

Appendix D: SMBus Details

9

10

12_

13_

19

15

10

18

| 440 TEXAS INSTRUMENTS | API Reference for the USB2ANY API Specification |
| --- | --- |

Appendix E: USB2ANY Schematic

| +UV DI ti |  | OPIOEVADOR TO 10 GND ON |
| --- | --- | --- |
| RR OrBS5138 | +33V TI op |  |

GNO

C6
0.1yF

56 4/C34/A4
PS 5/C35/A5
P6.6/CBGA6

4 	Pi3.7/0B7/A)

P7.0.ICBS/Al2
8- P7.1/039/A1

7	Pr.R.VB101A14 8- P7.3,0311/A15

P.5.0/A8NREF+NeREF+

P5.1/A9NREF.NeREF-

AVCC1

P5.4fAN

P5.5/XOUT

AM,

781

P82

DVCCI
DVSS1

P7.7r3OCLIVMCLK

P7.4.7B0.2 P5.7.750.1 PS.6r30.0 P4.7/PM_NONE PO.O/PM_NONE P4.5IPICLICA1RXD/PNIJICA1501v11 P4.4/PNI_LICA1TXD/PIA_UCA151X10 OVCC.2 DVSS2 P4.3/PM_LI031CLK/PM_LICA1STE P4.2/PICUCB1SOMIIPM_UCB1SCL P4.1/PAAUCB1SIMORALUCB1SDA 54.0/P MLICB15TE/PIALICAICLK P3.7/1130011TH/SVMOUT P3.6/1130.6 73 5M30 5 P3.4.0U0APORAD/LICAOSOMI

0

EXT5V FAULT

-( GPI PI 0 IyUART(RX0

(OPIO4ISPI(SIMOVUART(TXD

+33V

50

45	0.1v
47

CND

42

GPIWII2C(SDAI

mSr430;5529m,

SOLSOD

100

.70	0.47

3

o

p

Z. 2

E 4 4 4 4 4	4 4

OE.o.EZO-O-O-E7222222222222

I	4i mi~I AId 4

| CO P 0 | 2ICLOC |  |
| --- | --- | --- |
| °PIMP |  | )_ |

81	I	I

USB2ANY Schematic – Page 1

| litipTEXAS INSTRUMENTS | API Reference for the USB2ANY API Specification |
| --- | --- |

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | <VBUS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 |  |  |  |  |  |  |  |  | R4 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DM |
|  |  |  | 3 |  |  |  |  |  |  |  |  | 3386 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | < DP |  |  |
|  |  |  | 4 |  |  |  |  |  |  |  |  | 33 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | R7 1.5k |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PUR |
|  |  |  |  |  |  |  |  | 8 8 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SI |  |  |  |
|  |  | R8 |  |  |  |  |  |  |  | =C9 0.1pF |  |  |  | #1.2Meg T220pF R22 BSL fvusB C10 GND |  |  |  |  |
|  |  | 33k - GND |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | L3 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | TI'LM:00-11MY GND |  |  |  |  |  |  |  |  |  |  |  |  |

GND

R16

R17

GND

R20 w.

R19 1.00k

1.00k

GND

U711

R21

390

ADC1 >_

"I'L V2442

< V18 l

C16 T220pF

GND

| 1.5k |  |  |  |
| --- | --- | --- | --- |
| U6 +UV |  |  |  |
|  | DAC557 I IDBV |  |  |
|  | AO VOUT SCL GND |  |  |
| 5 |  |  | DAC0> |
| 4 |  |  |  |
|  | SDA VDD |  |  |

C18 T0.1pF

GN0

+3.3V

I

i1.5

'C21

T0.15pF GND

AIN B 1 >

SCLBRD>_

SDABR1I_

US DAC557 I I DIIV

+3.3V

DAC1>

5

4

GRID

--C20 T0.1pF

GND

AO	VOUT

SCL	GND

SDA	VDD

C19 T0.15pF

GND

GND

+3.3V

GND

USB2ANY Schematic – Page 2

101

litipTEXAS INSTRUMENTS

API Reference for the USB2ANY API Specification

TP7 ir

X1	X2	X3	X4

TP2 TP3 II

GND	GND

D2

I SM B5922BTS

1C13 22pF

GND

VBUS )

1	2

TP1	1.2

TP52553- I

2

IN	OUT

GND	ILIM

EN	-FAULT

GND

R5 100k

GND

TP4	U5

TPS2553- I

4.14 TPS73533DRB

+3.3V

I

GND

C12I 2.2pFT

GND

-C15 TO.01pF

GN- D

5

IN	OUT

GND	ILIM

EN	-FAULT

6	+5V EXT

5▪ C8

4	p F

130k T

GN- D

GND

EXT5V EN	>

EXT5V FAULT >

OUT

NC	NC 2

NC	C2 NR/FB 3

EN	GND

i GND

GN- D

GND

EXT3.3V FAULT

4

7*-GND

EXT3.3V EN

+3

3V EXT

7

6

| +3.3V EXT |  |  |  |  | Q4 RSS223PW |  | 8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R11 100k |  | Q2 BSS223PW |  |  |  |  |  |  |
|  |  |  |  |  |  | I DACO |  |  |
|  |  |  |  |  |  |  |  | O1O O 10 0r0 -O f() |
|  |  |  |  |  |  | EFC0/GPIO12/CLOCK |  |  |
|  |  |  |  |  |  | +5V EXT 6 |  |  |
|  |  |  |  |  |  | I +5V EXT |  |  |
|  |  |  |  |  |  | PWM3 |  |  |
| Q3 2N7002 W-74 GND |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | DAC1 > |  |  |
|  |  |  |  |  |  |  | GND |  |
|  |  |  |  |  |  |  |  | J3 |
|  |  |  |  |  |  | GPI07/PWMO |  |  |
|  |  | R12 1.5k |  | R13 1.5k |  |  |  |  |
|  |  |  |  |  |  | GPiOS/Pwmi/SPi(CS)-- |  |  |
|  |  |  |  |  |  |  |  | Or0 |
|  |  |  |  |  |  | GPI05/SPI(SOMI)/UART(RXD) |  |  |
|  |  |  |  |  |  |  |  | -O FO Or0 O r0 -O r0 |
|  |  |  |  |  |  | GPI04/SPI(SIMO)/UART(TXD)) |  |  |
|  |  |  |  |  |  | +3.3V EXT |  |  |
|  |  |  |  |  |  | I +3.3V EXT |  |  |
|  |  |  |  |  |  | I GPI03/PWM2 > |  |  |
|  |  |  |  |  |  | GPI02/SPI(SCLK) |  |  |
|  |  |  |  |  |  |  | GND |  |
| GPIO1/12C(SCL) |  |  |  |  |  |  |  | 14 |
| GPIO0/I2C(SDA) |  |  |  |  |  |  |  |  |

< ADCO

h_

< ADC1

GPI09/ADC2

GPI08/ADC3

GPI011NEREF

Or0

O1O

C14 .11pF

•• GND

GND

O r0

010

FBI

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

Appendix F: USB2ANY Cable Connections

The USB2ANY has four interface connectors: one USB 2.0 connector (J2) and three I/O connectors (J3, J4, and J5). The USB connector is a standard ‘A’ type mini USB receptacle. The I/O connectors are standard dual-row, .1” center, pin headers.

I/O connectors J3 and J5 are 8-pin type and J4 is a 10-pin type. They are configured such that they will accept either individual cable connections or a single 30-pin connection.

The standard USB2ANY Kit (HPA665-001) includes both a 10-pin cable and a 30-pin cable. The 10-pin cable is intended to be connected to J4. This single connection will supply the needs for many users as it provides access to the following interfaces and signals:

- I2C: SDA and SCL

- SPI: SCLK, MOSI, MISO, and CS

- UART: RX and TX

- Power: +3.3V External

- GPIO: GPIO0, GPIO1, GPIO2, GPIO3, GPIO4, GPIO5, GPIO6, and GPIO7

- PWM: PWM0, PWM1, and PWM2

- OneWire: OW1, OW2, and OW3

- µWire: SCLK, MOSI, MISO, and CS

- RFFE: SCLK and SDATA

- Interrupts: INT0, INT1, and INT2

103

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

The 10-pin cable is about 6 inches in length and has a keyed female 10-pin IDC connector on each end. The cable should be connected to the USB2ANY board as shown in the following picture (note that the key must be facing up, away from the board):

The opposite end of the cable is intended to be connected to the EVM (Evaluation Module) or other target device. The red stripe on the cable indicates Pin 1.

| Schematic Pin # | Cable Pin # | Signals Available |
| --- | --- | --- |
| J4-10 | 1 | GPIO7, PWM0, INT2, OW2, OW5 |
| J4-9 | 2 | GPIO6, PWM1, RFFE:SCLK, SPI:CS, INT1, µWIRE:CS, OW1 |
| J4-8 | 3 | GPIO5, SPI:SOMI, UART:RXD, µWIRE:SOMI |
| J4-7 | 4 | GPIO4, SPI:SIMO, UART:TXD, µWIRE:SIMO |
| J4-6 | 5 | +3.3VEXT |
| J4-5 | 6 | GND |
| J4-4 | 7 | GPIO3, PWM2, RFFE:SDATA, INT0 |
| J4-3 | 8 | GPIO2, ES:DOUT, SPI:SCLK, µWIRE:SCLK |
| J4-2 | 9 | GPIO1, I2C:SCL, OW3 |
| J4-1 | 10 | GPIO0, I2C:SDA, OW4 |

10-pin Cable Pinouts

IMPORTANT NOTE: The pin numbers for J4 on the schematics are for reference only and do not correspond to the pin numbers for cable connections.

104

105

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

The 30-pin cable is also about 6 inches in length and has a keyed female 30-pin IDC connector on each end. This cable provides access to all available signals. The cable should be connected to the USB2ANY board as shown in the following picture (note that the key must be facing up, away from the board):

The opposite end of the cable is intended to be connected to the EVM (Evaluation Module) or other target device. The red stripe on the cable indicates Pin 1.

|  | API Reference for the USB2ANY API Specification |
| --- | --- |

| Schematic Pin # | Cable Pin # | Signals Available |
| --- | --- | --- |
| J3-8 | 1 | DAC0 |
| J3-7 | 2 | PIO12, CLOCK, EFC0, INT3 |
| J3-6 | 3 | +5VEXT |
| J3-5 | 4 | GND |
| J3-4 | 5 | PWM3 |
| J3-3 | 6 | GND |
| J3-2 | 7 | DAC1 |
| J3-1 | 8 | GND |
|  | 9 | (unused) |
|  | 10 | (unused) |
| J4-10 | 11 | GPIO7, PWM0, INT2, OW2, OW5 |
| J4-9 | 12 | GPIO6, PWM1, RFFE:SCLK, SPI:CS, INT1, µWIRE:CS, OW1 |
| J4-8 | 13 | GPIO5, SPI:SOMI, UART:RXD, µWIRE:SOMI |
| J4-7 | 14 | GPIO4, SPI:SIMO, UART:TXD, µWIRE:SIMO |
| J4-6 | 15 | +3.3VEXT |
| J4-5 | 16 | GND |
| J4-4 | 17 | GPIO3, PWM2, RFFE:SDATA, INT0 |
| J4-3 | 18 | GPIO2, ES:DOUT, SPI:SCLK, µWIRE:SCLK |
| J4-2 | 19 | GPIO1, I2C:SCL, OW3 |
| J4-1 | 20 | GPIO0, I2C:SDA, OW4 |
|  | 21 | (unused) |
|  | 22 | (unused) |
| J5-8 | 23 | ADC0 |
| J5-7 | 24 | ADC1 |
| J5-6 | 25 | GPIO9, ADC2, ES:AIN |
| J5-5 | 26 | GPIO8, ADC3 |
| J5-4 | 27 | GND |
| J5-3 | 28 | GND |
| J5-2 | 29 | GPIO11, VREF+ |
| J5-1 | 30 | GPIO10, VREF- |

30-pin Cable Pinouts

IMPORTANT NOTE: The pin numbers for J3, J4, and J5 on the schematics are for reference only and do not correspond to the pin numbers for cable connections.

106